Chemical Division

| Sl No. | Product Name and Standards | Test Parameters | Standard Limit | Methods | Testing fee as per test parameter | Duration of test (Normal) | Total Fee (Normal) | Duration of test (Urgent) | Total Fee (Urgent) | Without fee |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1(1) Coconut Oil (Refined Grade ) (BDS 99:2007 ) | Description | The material shall be clear and free from sediment, suspended and other foreign matter, and separated water. | BDS 99:2007 : CL.- 4.1 | 100.00 | 14 | 3000.00 | 4 | 6000.00 | 0.00 |
|  |  | Moisture, volatile matter and insoluble impurities,% m/m | 0.10 (Max) | BDS 1584:1997 | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Colour in a 2.5 cm cell on lovibond scale expressed as Y+5 R | Not deeper than 2.0 | BDS 1584:1997 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Refractive index at 40oC | 1.4480-1.4500 | BDS 1584:1997, Cl.No.-7 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Specific gravity at 30/30 oC | 0.915-0.920 | BDS 1584:1997 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Saponification value, mg/g | 248-265 | BDS 1584:1991 Cl.No-15 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Iodine value (wijs) | 6-11 | BDS 1584:1991 Cl.No-19 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Acid value, mg/g | 0.60 (Max) (1.5 for perfumed, Max) | BDS 1584:1997 Cl.No-11 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Unsaponifiable matter,%m/m | 0.50 (Max) | BDS 1584:1997 Cl.No-17 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Melting point oC | 24-27 | BDS 1584:1997 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Titre value oC | 20-24 | BDS 1584:1997 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Polenske value | 13-18 | BDS 874 App-G | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Reichert value | 6-8.50 | BDS 874 App-F | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Flash point, pensky martens (Open) oC | ------ | BDS 99:2007, App-B | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Peroxide value (milliequivalents oxygen/ kg ) | 10.00 (Max) | BDS 99:2007, App-C | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Iron (as Fe),mg/kg | 1.50 (Max) | BDS 909, App-A | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Copper (Cu),mg/kg | 0.10 (Max) | BDS 909, App-B | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Lead (Pb),mg/kg | 0.10 (Max) | BDS 909, App-C | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Arsenic (As),mg/kg | 0.10 (Max) | BDS 909, App-D | 300.00 | 14 |  | 4 |  | 0.00 |
|  | 1(2)Coconut Oil (Crude Grade ) (BDS 99:2007 ) | Description | The material shall be clear and free from sediment, suspended and other foreign matter, and separated water. | BDS 99:2007 : CL.- 4.1 | 100.00 | 14 | 3000.00 | 4 | 6000.00 | 0.00 |
|  |  | Moisture, volatile matter and insoluble impurities, % m/m | 0.5 (Max) | BDS 1584:1997 CL.- 9 | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Colour in a 2.5 cm cell on lovibond scale expressed as Y+5 R | Not deeper than 10 | BDS 1584:1997 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Refractive index at 40 oC | 1.4480-1.4500 | BDS 1584:1997, Cl.No.-7 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Specific gravity at 30/30 oC | 0.915-0.920 | BDS 1584:1997 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Saponification value, mg/g | 248-265 | BDS 1584:1991 Cl.No-15 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Iodine value (wijs) | 6-13 | BDS 1584:1991 Cl.No-19 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Acid value, mg/g | 10.0 (Max) | BDS 1584:1997 Cl.No-11 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Unsaponifiable matter, %m/m | 1.0 (Max) | BDS 1584:1997 Cl.No-17 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Melting point oC | 24-27 | BDS 1584:1997 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Titre value oC | 20-24 | BDS 1584:1997 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Polenske value | 13-18 | BDS 874 App-G | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Reichert value | 6-8.50 | BDS 874 App-F | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Flash point, pensky martens (Closed) oC. | 100 (Min) | BDS 99:2007, App-B | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Peroxide value (milliequivalents oxygen/ kg ) | 10.00 (Max) | BDS 99:2007, App-C | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Iron (as Fe),mg/kg | 1.50 (Max) | BDS 909, App-A | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Copper (Cu),mg/kg | 0.10 (Max) | BDS 909, App-B | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Lead (Pb),mg/kg | 0.10 (Max) | BDS 909, App-C | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Arsenic (As),mg/kg | 0.10 (Max) | BDS 909, App-D | 300.00 | 14 |  | 4 |  | 0.00 |
| 2. | Duplicating Ink: ---- |  |  |  |  | 14 | 1200.00 | 4 | 2400.00 | 0.00 |
| 3. | Stencil Paper: ----- |  |  |  |  | 14 | 1300.00 | 4 | 2600.00 | 0.00 |
| 4. | Writing & Printing Papers (BDS 405:2012) | Description | Writing and printing papers shall be of uniform formation evenly finished fully sized and generally free from specks, holes and other blemishes. | BDS 405:2012. | 100.00 | 14 | 1200.00 | 4 | 2400.00 | 0.00 |
|  |  | Mechanical pulp | Not more than 20% | BDS 441:1991, P-4 | 900.00 | 14 |  | 4 |  | 0.00 |
|  |  | pH value | Not less than 5.0 | BDS 441:1991, P-29 | 200.00 | 14 |  | 4 |  | 0.00 |
| 5. | 5.a) Toilet Soap (Grade-1) (BDS 13:2021) | Total fatty matter, %m/m | 76.00 (Min) | BDS 406, Clause-9. | 800.00 | 14 | 4600.00 | 4 | 9200.00 | 0.00 |
|  |  | Rosin acid, %m/m | 2.00 (Max) | BDS 406, Clause-14 | 700.00 | 14 |  | 4 |  | 0.00 |
|  |  | Free caustic alkali (as NaOH), %m/m | 0.05 (Max) | BDS 406, Clause-6.2 | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Matter insoluble in alcohol, %m/m | 2.00 (Max) | BDS 406, Clause-5 | 600.00 | 14 |  | 4 |  | 0.00 |
|  |  | Chlorides (as NaCl) , %m/m | 1.00 (Max) | BDS 406, Clause-10 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Free carbonated alkali, %m/m | 0.50 (Max) | BDS 13:1994, Apx-A | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Lather in ml | 280 (Min) | IS 13498:1997 Annex-B | 500.00 | 14 |  | 4 |  | 0.00 |
|  | 5.b)Toilet Soap (Grade-2) (BDS 13:2021) | Total fatty matter, %m/m | 70.00 (Min) | BDS 406, Clause-9. | 800.00 | 14 | 4600.00 | 4 | 9200.00 | 0.00 |
|  |  | Rosin acid, %m/m | 3.00 (Max) | BDS 406, Clause-14 | 700.00 | 14 |  | 4 |  | 0.00 |
|  |  | Free caustic alkali (as NaOH), %m/m | 0.05 (Max) | BDS 406, Clause-6.2 | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Matter insoluble in alcohol, %m/m | 10.00 (Max) | BDS 406, Clause-5 | 600.00 | 14 |  | 4 |  | 0.00 |
|  |  | Chlorides (as NaCl) , %m/m | 1.50 (Max) | BDS 406, Clause-10 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Free carbonated alkali, %m/m | 1.00 (Max) | BDS 13:1994, Apx-A | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Lather in ml | 240 (Min) | IS 13498:1997 Annex-B | 500.00 | 14 |  | 4 |  | 0.00 |
| 6. | Fero galoTannel Fountain pen Ink: ---- |  |  |  |  | 14 | 2600.00 | 4 | 5200.00 | 0.00 |
| 7. | 7.(1)Stamp Pad Ink (BDS 90:2011) | Description | The material shall be in a fluid condition, in any one of the five colors violet, blue, green, red & black. It shall be free from undissolveed matter | BDS 90/2011 | 100.00 | 14 | 1200.00 | 4 | 2400.00 | 0.00 |
|  |  | Glycerin content, % v/v | 35-45 | BDS 90/2011,App-A | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Color | The color of ink shall match with the appropriate reference ink | BDS 90/2011,App-B | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Foreign matter | The material shall contain no saccharous material, gums or other foreign matter. | BDS 90/2011, App-B | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Performance test | i)10 impressions shall be clear ii) There shall be no offset of ink when a piece of writing paper stamped with rubber stamp is folded immediately after stamping | BDS 90/1990,App-C | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Color fastness | The strips of the material shall be no more affected in respect of resistance to light than those of the reference ink made from the appropriate dye in concentration. | BDS 90/1990,App-D | 300.00 | 14 |  | 5 |  | 0.00 |
|  | 7(2) Stamp Pad Ink (Grade-2) (BDS 90/2011) | Description | The material shall be in a fluid condition, in any one of the five colors violet, blue, green, red & black. It shall be free from undissolved matter | BDS 90/2011 | 100.00 | 14 | 1200.00 | 4 | 2400.00 | 0.00 |
|  |  | Glycerin content, % v/v | 35-45 | BDS 90/2011, App-A | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Color | The color of ink shall match with the appropriate reference ink | BDS 90/2011, App-B | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Foreign matter | The material shall contain no saccharous material, gums or other foreign matter. | BDS 90/2011, App-B | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Performance test | i)10 impressions shall be clear ii) There shall be no offset of ink when a piece of writing paper stamped with rubber stamp is folded immediately after stamping | BDS 90/1990, App-C | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Color fastness | The strips of the material shall be no more affected in respect of resistance to light than those of the reference ink made from the appropriate dye in concentration. | BDS 90/1990, App-D | 300.00 | 14 |  | 4 |  | 0.00 |
| 8. | Carbolic Soap (BDS 181:2001) | Total fatty matter, %m/m | 65.00 (Min) | BDS 12:1987, App-B | 500.00 | 14 | 2200.00 | 4 | 4400.00 | 0.00 |
|  |  | Free caustic alkali (as Na2O), %m/m | 0.05 (Max) | BDS 12:1987, App-F | 400.00 | 14 |  | 4 |  | 0.00 |
|  |  | Total free alkali (as Na2O), %m/m | 0.30 (Max) | BDS 13:1987, App-H | 400.00 |  |  | 4 |  | 0.00 |
|  |  | Matter insoluble in Ethanol, %m/m | 2.00 (Max) | BDS 12:1987, App-E | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Chlorides (as NaCl) , %m/m | 1.00 (Max) | BDS 12:1987, App-G | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Phenolic substance, %m/m | 1.0 – 2.0 | BDS 181:2001, App-A | 300.00 | 14 |  | 4 |  | 0.00 |
| 9. | Ready Mixed paint, brushing, Finishing, Semi gloss for general purposes (BDS 402 :1989,With amendment 1, 2 :2007 ) | Drying time |  |  | 0 | 14 | 2900.00 | 4 | 5800.00 | 0.00 |
|  |  | a) Surface dry | Between 4 to 12 hours Not | Cl No. 8. 3BDS 113 : 1960 | 100 | 14 |  | 4 |  | 0.00 |
|  |  | b) Hard dry | more than 24 hours | Cl No. 8.5BDS 113 : 1960 | 100 | 14 |  | 4 |  | 0.00 |
|  |  | c) Tack free | Not more than 48 hours | Cl No. 8. 6BDS 113 : 1960 | 100 | 14 |  | 4 |  | 0.00 |
|  |  | Consistency | Smooth and uniform | Cl No. 8. 8BDS 113 : 1960 | 100 | 14 |  | 4 |  | 0.00 |
|  |  | Colour | Close match to BS colour No. 105 or 106 | Cl No. 9BDS 113 : 1960 | 100 | 14 |  | 4 |  | 0.00 |
|  |  | Flash point, °C, Min | 35 | Cl No. 17BDS 113 : 1960 | 500 | 14 |  | 4 |  | 0.00 |
|  |  | Gloss | Shall dry with a full gloss after 24 hours | Annex B of BDS 402:2007 | 100 | 14 |  | 4 |  | 0.00 |
|  |  | Opacity | Not less than 75 percent of the approved sample | Cl No. 25BDS 113 : 1960 | 100 | 14 |  | 4 |  | 0.00 |
|  |  | Flexibility and adhesion | No visible damage or detachment | Cl No. 10BDS 113 : 1960 | 100 | 14 |  | 4 |  | 0.00 |
|  |  | Scratch resistance | No such scratch as to show the bare metal | Cl No. 26BDS 113 : 1960 | 100 | 14 |  | 4 |  | 0.00 |
|  |  | Resistance to white spirit | Shall not show of softening or removal | Cl No. 13BDS 113 : 1960 | 100 | 14 |  | 4 |  | 0.00 |
|  |  | Resistance to kerosene | Shall not show signs of softening or removal | Cl No. 11BDS 113 : 1960 | 100 | 14 |  | 4 |  | 0.00 |
|  |  | Viscosity | shall not show hard settling objectionable skinning or tendency for the medium to gel when examined in Ford Cup ( Flow Cup ) Viscometer No. 4 | - | 400 | 14 |  | 4 |  | 0.00 |
|  |  | Fastness to light | Not inferior to the approved sample | Appendix B-3BDS 928 : 1980 | 100 | 14 |  | 4 |  | 0.00 |
|  |  | Finish | Smooth and semi-gloss | Cl No. 9BDS 113 : 1960 | 200 | 14 |  | 4 |  | 0.00 |
|  |  | Water content (if water is suspected to be present), percent by mass, max | 0.5 | Cl No. 23BDS 113 : 1960 | 400 | 14 |  | 4 |  | 0.00 |
|  |  | Protection against corrosion under conditions of condensation | No signs of break down or corrosion | Cl No. 11BDS 113 : 1960 | 100 | 14 |  | 4 |  | 0.00 |
|  |  | Mass in kg per 10 litre | Within 3 percent of the approved sample | Appendix B-5BDS 928 : 1980 | 100 | 14 |  | 4 |  | 0.00 |
| 10. | Shoe Polish (BDS 1006:2006) | Description |  |  | 100.00 | 14 | 2000.00 | 5 | 4000.00 | 0.00 |
|  |  | a) Colour | The material shall be black tan (light brown) dark tan (dark brown) or neutrally matching the colour of the leather foot wear or in any colour as agreed to between the purchaser and the supplier in case to bulk contract. | BDS: 1006:2006 | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Odour | The material shall not have a disagreeable odour. | BDS: 1006:2006 | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | c) Polishing property | The polish shall be smooth spreading on the upper shoe leather and on gentle rubbing with brush or polishing cloth the gloss shall appear. | BDS: 1006:2006 | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | d) Colour of the water extract | When stirred with water hot enough to melt the polish, the water layer shall not have more than of faint coloration. | BDS: 1006:2006 | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | Consistency | The material shall be smooth, homogeneous & semi solid mass, free from gritty material. It shall not flow at ordinary temperatures. | BDS: 1006:2006 App-B | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Applicability | The polish shall not crumble or dry too rapidly and shall not produce a non tacky polished surface without coloring in the leather unduly. | BDS: 1006:2006, App-C | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Softening point of nonvolatile matter: |  | BDS: 1006:2006, App-D | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | a) Initial oC | 60.00 (Min) |  | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Final oC | 70.00 (Min) |  | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | Ash of the non volatile matter, %m/m | 1.50 (Max) | BDS: 1006:2006, App-E | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH of the water extract. | 6.5 to 9.0 | BDS: 1006:2006, App-F | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Distillation range of volatile portion | 140 to 220 oC | BDS: 1006:2006, App-G | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Flash point of volatile portion | 35oC (Min) | BDS: 1006:2006, App-G | 500.00 | 14 |  | 5 |  | 0.00 |
| 11. | 11.a) Laundry Soap (Type-3, Filled) (BDS 12:2019) | Description | The sample shall be a well saponified product in the form of tablet or bar, uniform in colour, of firm and texture, free from dirt and fragmentation. It shall have good lathering and cleansing properties. | BDS 12:2019 | 100.00 | 14 | 5900.00 | 5 | 11800.00 | 0.00 |
|  |  | Total fatty matter, %m/m | 40.00 (Min) | Cl. No in BDS 406: 1990- 9 | 1600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Free caustic alkali (as NaOH), %m/m | 0.30 (Max) | Cl. No in BDS 406: 1990- 6 | 1600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Unsaponified fatty matter, %m/m | 1.0(Max) | Cl. No in BDS 406: 1990- 13 | 1600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Chlorides (as NaCl) , %m/m | 3.00 (Max) | Cl. No in BDS 406: 1990- 10 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  | 11.b) Laundry Soap (Type-2, Built) (BDS 12:2019) | Description | The sample shall be a well saponified product in the form of tablet or bar, uniform in colour, of firm and texture, free from dirt and fragmentation. It shall have good lathering and cleansing properties. | BDS 12:2019 | 100.00 | 14 | 5900.00 | 5 | 11800.00 | 0.00 |
|  |  | Total fatty matter, %m/m | 52.00 (Min) | Cl. No in BDS 406: 1990- 9 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Free caustic alkali (as NaOH), %m/m | 0.20 (Max) | Cl. No in BDS 406: 1990- 6 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Unsaponified fatty matter , %m/m | 0.40 (Max) | Cl. No in BDS 406: 1990- 13 | 1600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Chlorides (as NaCl) , %m/m | 1.00 (Max) | Cl. No in BDS 406: 1990- 10 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Matter insoluble in alcohol, %m/m | 15.00 (Max) | Cl. No in BDS 406: 1990- 8 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Matter insoluble in water, %m/m | 10.00 (Max) | Cl. No in BDS 406: 1990- 7 | 600.00 | 14 |  | 5 |  | 0.00 |
|  | 11.c) Laundry Soap(Type-2, Pure) (BDS 12:2019) | Description | The sample shall be a well saponified product in the form of tablet or bar, uniform in colour, of firm and texture, free from dirt and fragmentation. It shall have good lathering and cleansing properties. | BDS 12:2019 | 100.00 | 14 | 5900.00 | 5 | 11800.00 | 0.00 |
|  |  | Total fatty matter, %m/m | 61.00 (Min) | Cl. No in BDS 406: 1990- 9 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Free caustic alkali (as NaOH), %m/m | 0.10 (Max) | Cl. No in BDS 406: 1990- 6 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Unsaponified fatty matter , %m/m | 0.50 (Max) | Cl. No in BDS 406: 1990- 13 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Chlorides (as NaCl) , %m/m | 1.00 (Max) | Cl. No in BDS 406: 1990- 10 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Matter insoluble in alcohol, %m/m | 2.50 (Max) | Cl. No in BDS 406: 1990- 8 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Matter insoluble in water, %m/m | ----- | Cl. No in BDS 406: 1990- 7 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Titre of total fatty acid,oC, Min | 33.0 (Min) | Cl. No in BDS 406: 1990- 16 | 600.00 | 14 |  | 5 |  | 0.00 |
| 12. | Alquatra (BDS 69 (Pt-2):2006) | Description | The material shall be homogeneous black solution type paint. It shall be free from dirt. | BDS 69(Pt-2):2006 | 100.00 | 14 | 1700.00 | 5 | 3400.00 | 0.00 |
|  |  | Consistency | The paint film after drying shall give a continuous uniform film, without sagging on the unpainted section. | BDS 69(Pt-2):2006 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Permeability and resistance to salt water | The film of alquatra shall show no discoloration or softening and the surface of the panel shall show no pitting or corrosion. | BDS 69(Pt-2):2006 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Drying time, hard dry | 48 hours (Max) | BDS 1013:1960, Cl-9 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Finish and application properties | Smooth and glossy and to pass the test. | BDS 1013:1960, App-C | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Water content, % m/m | 0.5(Max) | BDS 1013:1960, Cl-24 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Flexibility and adhesion after 49 hours air drying | No visible change or detachment of film. | BDS 1013:1960, Cl-11 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Flash point, 0C | Not below 30 0C | BDS 1013:1960, Cl-18 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Viscosity at 30 0C (in second) | 35-55 | BDS 1013:1960, App-D | 200.00 | 14 |  | 5 |  | 0.00 |
| 13. | Newsprint Paper (BDS 845:2012) | Description | Newsprint paper shall be free from shives, pin holes, slime holes, stock lumps, wrinkles and calender cuts. | BDS 845:2012. | 100.00 | 14 | 1200.00 | 4 | 2400.00 | 0.00 |
|  |  | Moisture, % m/m | 8.0 (Max) | BDS 845:2012. | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Mechanical pulp | Min 65% | BDS 441:1991 | 900.00 | 14 |  | 4 |  | 0.00 |
| 14. | 14(1)Urea Fertilizer (Type: Prilled) (BDS 217:2011) | Description | The material shall be in the form of prills, pellets or shots. It shall be free from visible impurities and from dust. | BDS 217:1993 | 100.00 | 14 | 2000.00 | 4 | 4000.00 | 0.00 |
|  |  | Particle size: The portion of the material passing through 3.4 mm BDS sieve and retained on 1mm BDS sieve | shall not be less than 90 percent by mass | BDS 217: 2011; Appendix B | 600.00 | 14 |  | 4 |  | 0.00 |
|  |  | Moisture, %m/m | 1.00(Max) | MFA | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Total Nitrogen, %m/m | 46.0(Min) | MFA | 700.00 | 14 |  | 4 |  | 0.00 |
|  |  | Biuret, %m/m | 1.5 (Max) | MFA | 400.00 | 14 |  | 4 |  | 0.00 |
|  | 14(2)Urea Fertilizer (Type: Granular) (BDS 217:2011) | Description | The material shall be in the form of prills, pellets or shots. It shall be free from visible impurities and from dust. | BDS 217:1993 | 100.00 | 14 | 2000.00 | 4 | 4000.00 | 0.00 |
|  |  | Particle size: The portion of the material retained on 4.75 mm BDS sieve | shall not be more than 10 percent by mass | BDS 217: 2011; Appendix B | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | the portion of the material retained on 2 mm BDS sieve | shall not be less than 93 percent by mass |  | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Moisture, %m/m | 0.50 (Max) | MFA | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Total Nitrogen, %m/m | 46.0 (Min) | MFA | 700.00 | 14 |  | 4 |  | 0.00 |
|  |  | Biuret, %m/m | 1.5 (Max) | MFA | 400.00 | 14 |  | 4 |  | 0.00 |
| 15. | 15(1) Aluminium Sulphate, Non-Ferric ( TECH Grade ) (BDS 149: 2014 (2nd Rev.) | Description | The material shall be in the form of white, slightly deliquescent crystals or granules. | BDS 149: 2014 (2nd Rev.) | 55.00 | 14 | 1055.00 | 4 | 2110.00 | 0.00 |
|  |  | Insoluble matter, per cent by mass, Max | 0.30 | BDS 149: 2014, App A | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | pH ( 5 per cent solution), | 2.8 (Min) | BDS 149: 2014, App B | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Iron (as Fe), per cent by mass | 0.001 (Max) | BDS 149: 2014, App D | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Aluminium (as Al2O3) percent by mass | 17.00 (Min) | BDS 149: 2014, App H | 300.00 | 14 |  | 4 |  | 0.00 |
|  | 15(2) Aluminium Sulphate, Non-Ferric (Pure Grade) (BDS 149: 2014 (2nd Rev.) | Description | The material shall be in the form of white, slightly deliquescent crystals or granules. | BDS 149: 2014 (2nd Rev.) | 55.00 | 14 | 1055.00 | 4 | 2110.00 | 0.00 |
|  |  | Insoluble matter, per cent by mass, Max | 0.02 | BDS 149: 2014, App A | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | pH ( 5 per cent solution), | 2.9 (Min) | BDS 149: 2014, App B | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Heavy metals (as Pb), parts per million, Max | 40.00 | BDS 149: 2014, App C | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Iron (as Fe), per cent by mass Max | 0.004 | BDS 149: 2014, App D | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Arsenic (as As2O3), parts per million, Max | 8.00 | BDS 149: 2014, App F | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Ammonium salts (as NH3), per cent by mass, Max | 0.025 | BDS 149: 2014, App G | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Aluminium (as Al2O3) percent by mass | 16.60 – 17.60 | BDS 149: 2014, App H | 100.00 | 14 |  | 4 |  | 0.00 |
|  | 15(3) Aluminium Sulphate, Non-Ferric (AR Grade) (BDS 149: 2014 (2nd Rev.) | Description | The material shall be in the form of white, slightly deliquescent crystals or granules. | BDS 149: 2014 (2nd Rev.) | 55.00 | 14 | 1055.00 | 4 | 2110.00 | 0.00 |
|  |  | Insoluble matter, per cent by mass, Max | 0.005 | BDS 149: 2014, App A | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | pH ( 5 per cent solution), | 2.2 (Min) | BDS 149: 2014, App B | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Heavy metals (as Pb),ppm, Max | 10 | BDS 149: 2014, App C | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Iron (as Fe), per cent by mass | 0.002 ( Max) | BDS 149: 2014, App D | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Chlorides | To pass the test | BDS 149: 2014, App E | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Arsenic (as As2O3), ppm | 5.00 | BDS 149: 2014, App F | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Ammonium salts (as NH3),% by mass | 0.025 (Max) | BDS 149: 2014, App G | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Aluminium (as Al2O3) percent by mass | 16.6-17.6 | BDS 149: 2014, App H | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Sodium (as Na), percent by mass, Max | 0.05 | BDS 149: 2014, App J | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Potassium (as K) per cent by mass, Max | 0.01 | BDS 149: 2014, App K | 100.00 | 14 |  | 4 |  | 0.00 |
| 16. | TSP (Granular) (BDS 216:1991,Amendment No 1: 2007) | Description | The material shall be free from excessive lumps and shall not form hard cakes on storage. | BDS 216:2007 | 100.00 | 14 | 2200.00 | 4 | 4400.00 | 0.00 |
|  |  | Total phosphate (as P2O5), percent by weight, Min | 46.0 | BDS 216:2007 Appendix - D | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Water soluble phosphate (as P2O5), percent by weight, Min | 40.0 | BDS 216:2007 Appendix - E | 400.00 | 14 |  | 4 |  | 0.00 |
|  |  | Free phosphoric acid (as P2O5), percent by weight, Max | 3.0 | BDS 216:2007 Appendix - C | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Moisture, percent by weight, Max | 5.0 | BDS 216:2007 Appendix - B | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Percent by mass of the material retained on 1.00 mm BDS sieve, Min | 85.0 | BDS 216:2007 Appendix - E | 500.00 | 14 |  | 4 |  | 0.00 |
| 17. | Ribons Typewriter |  |  |  |  | 14 | 1031.00 | 4 | 2062.00 | 0.00 |
| 18. | Sole leather (vegetable Tanned) (BDS 340:2012) | Apparent density, Min | 0.9 | BDS 591, LP:5 | 50 | 14 | 700.00 | 4 | 1400.00 | 0.00 |
|  |  | Water absorption, percent,Max: |  | BDS 591, LP:12 | 50 | 14 |  | 4 |  | 0.00 |
|  |  | a) in 30 minutes | 45 |  | 0 | 14 |  | 4 |  | 0.00 |
|  |  | b) in 2 hours | 45 |  | 0 | 14 |  | 4 |  | 0.00 |
|  |  | c) in 24 hours | 50 |  | 0 | 14 |  | 4 |  | 0.00 |
|  |  | Resistance to cracking | Shall not crack on mandrel No.1 and shall not show any sign of permanent loosening of the grain in the form of wrinkles. | BDS 591, LP:13 | 50 | 14 |  | 4 |  | 0.00 |
|  |  | Total ash, percent by mass, | 4.0 ( Max) | BDS 591, LC:3 | 100 | 14 |  | 4 |  | 0.00 |
|  |  | Solvent extractable substance, percent by mass, | 6 (Max) | BDS 591, LC:4 | 100 | 14 |  | 4 |  | 0.00 |
|  |  | water soluble matter,%by mass, | 21 (Max) | BDS 591, LC:6 | 50 | 14 |  | 4 |  | 0.00 |
|  |  | pH of water soluble | Not below 3.5 | BDS 591, LC:18 | 50 | 14 |  | 4 |  | 0.00 |
|  |  | Differential number. Max | 0.7 | BDS 591, LC:18 | 50 | 14 |  | 4 |  | 0.00 |
|  |  | Degree of Tannage, Min | 58 | BDS 591, LC:18 | 50 | 14 |  | 4 |  | 0.00 |
|  |  | Sulphated ash of water solubles, in percent, Max | 2 | BDS 591, LC:7 | 100 | 14 |  | 4 |  | 0.00 |
|  |  | Hide substance, % by mass, | - (Min) | BDS 591, LC:5 | 50 | 14 |  | 4 |  | 0.00 |
| 19. | 19(1) Carbon Paper, Grade-1 (BDS 411:2006) | Description | The sample shall be free from waves, wrinkles, tears & cuts. The side which is not coated shall be marked by printing to facilitate distinguishing it from the coated side. | BDS 411:2006 | 100.00 | 14 | 1300.00 | 4 | 2600.00 | 0.00 |
|  |  | Substance of base paper, g/m2 | 26 ± 1.5 | BDS 411:2006 | 600.00 | 14 |  | 4 |  | 0.00 |
|  |  | Coating (Without back coating), g/m2 | 12 (Min) | BDS 411:2006 | 600.00 | 14 |  | 4 |  | 0.00 |
|  | 19(2) Carbon Paper (BDS 411:2006) Grade-2 | Description | The sample shall be free from waves, wrinkles, tears & cuts. The side which is not coated shall be marked by printing to facilitate distinguishing it from the coated side. | BDS 411:2006 | 100.00 | 14 | 1300.00 | 4 | 2600.00 | 0.00 |
|  |  | Substance of base paper, g/m2 | 17 ± 1.0 | BDS 411:2006 | 600.00 | 14 |  | 4 |  | 0.00 |
|  |  | Coating ( Without back coating ), g/m2 | 11 (Min) | BDS 411:2006 | 600.00 | 14 |  | 4 |  | 0.00 |
|  | 19(3) Carbon Paper (BDS 411:2006) Grade-3 | Description | The sample shall be free from waves, wrinkles, tears & cuts. The side which is not coated shall be marked by printing to facilitate distinguishing it from the coated side. | BDS 411:2006 | 100.00 | 14 | 1300.00 | 4 | 2600.00 | 0.00 |
|  |  | Substance of base paper, g/m2 | 20.5 ± 1.5 | BDS 411:2006 | 600.00 | 14 |  | 4 |  | 0.00 |
|  |  | Coating ( Without back coating ), g/m2 | 12 (Min) | BDS 411:2006 | 600.00 | 14 |  | 4 |  | 0.00 |
|  | 19(4) Carbon Paper (BDS 411:2006) Grade-4 | Description | The sample shall be free from waves, wrinkles, tears & cuts. The side which is not coated shall be marked by printing to facilitate distinguishing it from the coated side. | BDS 411:2006 | 100.00 | 14 | 1300.00 | 4 | 2600.00 | 0.00 |
|  |  | Substance of base paper, g/m2 | 26 ± 1.5 | BDS 411:2006 | 600.00 | 14 |  | 4 |  | 0.00 |
|  |  | Coating ( Without back coating ), g/m2 | 14 (Min) | BDS 411:2006 | 600.00 | 14 |  | 4 |  | 0.00 |
| 20. | Tooth Paste (BDS 1216:2012) | Composition | Tooth-paste shall not contain mono or disaccharides (Sucrose and fermentable carbohydrates) | BDS 1216:2012 | 100.00 | 14 | 4300.00 | 5 | 8600.00 | 0.00 |
|  |  | Consistency | The material shall be a homogeneous mixture and free from lumps and discrete particles. | BDS 1216:2012 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Flavour | The flavour of the material will be distinct and pleasant. | BDS 1216:2012 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Irritation | The material shall not irritate the mucous membrane of the mouth. | BDS 1216:2012 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Safety | When used in normal manner the material shall not cause injury to the teeth, gums, mucous membrane of the mouth. | BDS 1216:2012 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Ease of extrusion, %m/m | 91.00 (Min) | BDS 1216:2012 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Homogeneity | The material shall be homogeneous at 27 ± 2 oC in the tube/container | BDS 1216:2012 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Stability | The material shall show no signs of deterioration at 45oC for 28 days and shall not have contaminated the tooth paste inside. | BDS 1216:2012 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Tube inertness | There will be no corrosion or other injury of the tube and cap by the material. | BDS 1216:2012 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Hard & sharp edged abrasive particles. | Absent | App-B | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Spreadibility ,cm | 8.50 (Max) | App-C | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Fineness, Particle retained on |  | App-D |  | 14 |  | 5 |  | 0.00 |
|  |  | a) 150m sieve, %m/m | 1.0 (Max) |  | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) 75m sieve, %m/m | 2.5 (Max) |  | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH of aqueous suspension | 5.50 -10.50 | App-E-1 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metal (as Pb),ppm | 20.00 (Max) | App-F | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3),ppm | 2.00 (Max) | App-G | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Foaming power, in ml | 50.00 (Min) | App-H | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Fluoride ion, ppm |  | App-I |  | 14 |  | 5 |  | 0.00 |
|  |  | a) For non fluoridated | 50.00 (Max) |  |  | 14 |  | 5 |  | 0.00 |
|  |  | b) For fluoridated | 1000.00 (Max) |  | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Microbial purity |  |  | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | a) Total aerobic bacterial per gram | 1000.00 (max) | App-K | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Salmonellae (could not be done) and Escheerichia coil per 10 grams. | Absent | App-K | 100.00 | 14 |  | 5 |  | 0.00 |
| 21. | Shampoo (BDS 1269:2021) | Description | The sample shall be in the form of a liquid, emulsion, paste, powder or bar. It may be colored and perfumed. | BDS 1269:2021 | 100.00 | 14 | 9900.00 | 5 | 19800.00 | 0.00 |
|  |  | pH at 27 ± 2 O C | 4.0 – 9.0 | BDS 1269:2021,App-B | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Active detergent content as SLES or its equivalent, %m/m | 3.00 (Min) | BDS 1269:2021, App-C | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Foam height for two percent solution | 150.0 mm (Min) | BDS 1269:2021, App-D | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), mg/kg | 2.0 (Max) | BDS 1333 | 800.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead (Pb), mg/kg | 20.0 (Max) | BDS 1465 | 800.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mercury (Hg), mg/kg | 1.0 (Max) | BDS ISO/TR 17276 | 800.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cadmium (Cd), mg/Kg | 3.0 (Max) | BDS ISO/TR 17276 | 800.00 | 14 |  | 5 |  | 0.00 |
|  |  | Aerobic Plate Count/mL | 1000 (Max) | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | i) Pseudomonas aeruginosa | Absent in 10 ml | BDS: 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | ii) Staphylococcus aureus | Absent in 10 ml | BDS: 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | iii) Escherichia coli | Absent in 10 ml | BDS: 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | iv) Candida albicans | Absent in 10 ml | BDS: 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
| 22. | Aluminium Sulphate (Fertilizer) |  |  |  |  | 14 | 1600.00 | 4 | 3200.00 | 0.00 |
| 23. | Ball point pen (BDS 1384:2022) | Flame resistance test | The plastic parts shall be nonflammable or flame resistance or slow burning. | BDS 1384:2022, CL-8.1 | 100.00 | 14 | 700.00 | 4 | 1400.00 | 0.00 |
|  |  | Corrosion resistance test | All the metal parts shall be corrosion resistant and shall pass corrosion resistance test. | BDS 1384:2022, CL-8.2 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Feathering and Penetration test | The writing shall not feather or spread and shall not have penetrated to the reverse side of the paper. | BDS 1384:2022, CL-8.11 | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Resistance to water test | Writing shall be legible, after the test. | BDS 1384:2022, CL-8.12 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Finish | Ball point pens shall have a smooth finish and shall have no sharp edges or feathers. | BDS 1384:2022, CL-5.0 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Usable ink content, % m/m | Not less than 90.00 | BDS 1384:2022, CL-8.14.4 | 100.00 | 14 |  | 4 |  | 0.00 |
| 24. | 24(1) Ceramic Tableware (Type- Flatware) (BDS 485:2000,Amend-2006 ) | Material | The tableware shall be fired ceramic ware covered by glaze properly matured and fused with the body. The body shall show upon fracture a dense homogeneous vitrified texture. The glaze shall be of uniform colour, impervious and as free as possible from visual defects. | BDS 485:2000 Cl-4.1 | 200.00 | 14 | 2900.00 | 5 | 5800.00 | 0.00 |
|  |  | Finish | Except for the resting surface (bottom rim) of the hollow ware, the entire surface of items of table ware shall be covered by a uniform and continuous, hard impervious glaze, which is cleanable and retain this quality in addition the resting surface of items other than hollow ware shall be non-abrasive in texture. The glazed surface shall be free from defects namely craze, Crawling patches and fingerprints. | BDS 485:2000, Cl-4.3 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Wet ability | Table ware used for eating purposes shall pass the test. | BDS 485:2000, Cl-5.3 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cadmium (Cd). released, mg/dm2 | 0.07 (Max) | ISO 6486-1: 2019(E) | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead (Pb), released, mg/dm2 | 0.80 (Max) | ISO 6486-1: 2019(E) | 1000.00 | 14 |  | 5 |  | 0.00 |
|  | 24(2) Ceramic Tableware (Type-Small Hollowware) (BDS 485:2000,Amend-2006 ) | Material | The tableware shall be fired ceramic ware covered by glaze properly matured and fused with the body. The body shall show upon fracture a dense homogeneous vitrified texture. The glaze shall be of uniform colour, impervious and as free as possible from visual defects. | BDS 485:2000 Cl-4.1 | 200.00 | 14 | 2900.00 | 5 | 5800.00 | 0.00 |
|  |  | Finish | Except for the resting surface (bottom rim) of the hollow ware, the entire surface of items of table ware shall be covered by a uniform and continuous, hard impervious glaze, which is cleanable and retain this quality in addition the resting surface of items other than hollow ware shall be non-abrasive in texture. The glazed surface shall be free from defects namely craze, Crawling patches and fingerprints. | BDS 485:2000, Cl-4.3 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Wet ability | Table ware used for eating purposes shall pass the test. | BDS 485:2000, Cl-5.3 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cadmium (Cd). released, mg/l | 0.05 (Max) | ISO 6486-1:2019(E) | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead (Pb), released, mg/l | 2.00 (Max) | ISO 6486-1:2019(E) | 1000.00 | 14 |  | 5 |  | 0.00 |
|  | 24(3) Ceramic Tableware (Type- Large Hollowware) (BDS 485:2000,Amend-2006 ) | Material | The tableware shall be fired ceramic ware covered by glaze properly matured and fused with the body. The body shall show upon fracture a dense homogeneous vitrified texture. The glaze shall be of uniform colour, impervious and as free as possible from visual defects. | BDS 485:2000 Cl-4.1 | 200.00 | 14 | 2900.00 | 5 | 5800.00 | 0.00 |
|  |  | Finish | Except for the resting surface (bottom rim) of the hollow ware, the entire surface of items of table ware shall be covered by a uniform and continuous, hard impervious glaze, which is cleanable and retain this quality in addition the resting surface of items other than hollow ware shall be non-abrasive in texture. The glazed surface shall be free from defects namely craze, Crawling patches and fingerprints. | BDS 485:2000, Cl-4.3 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Wet ability | Table ware used for eating purposes shall pass the test. | BDS 485:2000, Cl-5.3 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cadmium (Cd). released, mg/l | 0.25 (Max) | ISO 6486-1:2019(E) | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead (Pb), released, mg/l | 1.00 (Max) | ISO 6486-1:2019(E) | 1000.00 | 14 |  | 5 |  | 0.00 |
|  | 24(4) Ceramic Tableware (Type- Storage Hollowware) (BDS 485:2000,Amend-2006 ) | Material | The tableware shall be fired ceramic ware covered by glaze properly matured and fused with the body. The body shall show upon fracture a dense homogeneous vitrified texture. The glaze shall be of uniform colour, impervious and as free as possible from visual defects. | BDS 485:2000 Cl-4.1 | 200.00 | 14 | 2900.00 | 5 | 5800.00 | 0.00 |
|  |  | Finish | Except for the resting surface (bottom rim) of the hollow ware, the entire surface of items of table ware shall be covered by a uniform and continuous, hard impervious glaze, which is cleanable and retain this quality in addition the resting surface of items other than hollow ware shall be non-abrasive in texture. The glazed surface shall be free from defects namely craze, Crawling patches and fingerprints. | BDS 485:2000, Cl-4.3 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Wet ability | Table ware used for eating purposes shall pass the test. | BDS 485:2000, Cl-5.3 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cadmium (Cd). released, mg/l | 0.25 (Max) | ISO 6486-1:2019(E) | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead (Pb), released, mg/l | 0.50 (Max) | ISO 6486-1:2019(E) | 1000.00 | 14 |  | 5 |  | 0.00 |
|  | 24(5) Ceramic Tableware (Type- Cups and Mugs ) (BDS 485:2000,Amend-2006 ) | Material | The tableware shall be fired ceramic ware covered by glaze properly matured and fused with the body. The body shall show upon fracture a dense homogeneous vitrified texture. The glaze shall be of uniform colour, impervious and as free as possible from visual defects. | BDS 485:2000 Cl-4.1 | 200.00 | 14 | 2900.00 | 5 | 5800.00 | 0.00 |
|  |  | Finish | Except for the resting surface (bottom rim) of the hollow ware, the entire surface of items of table ware shall be covered by a uniform and continuous, hard impervious glaze, which is cleanable and retain this quality in addition the resting surface of items other than hollow ware shall be non-abrasive in texture. The glazed surface shall be free from defects namely craze, Crawling patches and fingerprints. | BDS 485:2000, Cl-4.3 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Wet ability | Table ware used for eating purposes shall pass the test. | BDS 485:2000, Cl-5.3 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cadmium (Cd). released, mg/l | 0.25 (Max) | ISO 6486-1:2019(E) | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead (Pb), released, mg/l | 0.50 (Max) | ISO 6486-1:2019(E) | 1000.00 | 14 |  | 5 |  | 0.00 |
|  | 24(6) Ceramic Tableware (Type- Cooking Ware) (BDS 485:2000,Amend-2006 ) | Material | The tableware shall be fired ceramic ware covered by glaze properly matured and fused with the body. The body shall show upon fracture a dense homogeneous vitrified texture. The glaze shall be of uniform colour, impervious and as free as possible from visual defects. | BDS 485:2000 Cl-4.1 | 200.00 | 14 | 2900.00 | 5 | 5800.00 | 0.00 |
|  |  | Finish | Except for the resting surface (bottom rim) of the hollow ware, the entire surface of items of table ware shall be covered by a uniform and continuous, hard impervious glaze, which is cleanable and retain this quality in addition the resting surface of items other than hollow ware shall be non-abrasive in texture. The glazed surface shall be free from defects namely craze, Crawling patches and fingerprints. | BDS 485:2000, Cl-4.3 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Wet ability | Table ware used for eating purposes shall pass the test. | BDS 485:2000, Cl-5.3 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cadmium (Cd). released, mg/l | 0.05 (Max) | ISO 6486-1:2019(E) | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead (Pb), released, mg/l | 0.50 (Max) | ISO 6486-1:2019(E) | 1000.00 | 14 |  | 5 |  | 0.00 |
| 25. | Synthetic Detergent Powder (BDS 1445:2019) | Description | The Product shall be a homogeneous, free flowing powder which is readily soluble in water. It shall be free from any unpleasant odour and shall possess good lathering and cleaning properties. | BDS 1445:2019 | 100.00 | 14 | 5000.00 | 4 | 10000.00 | 0.00 |
|  |  | Active ingredient content, % m/m | 10.0 (Min) | BDS1445:2003,APP-B-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Total phosphate content express as P2O5, % m/m | 5.00 (Max) | BDS1445:2003,APP-B-4 | 400.00 | 14 |  | 4 |  | 0.00 |
|  |  | pH of 1.0 % solution at 27±2 ˚C | 6.0-11.0 | BDS1445:2003,APP-B-5 | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Matter insoluble in water, %m/m | 20.0(Max) | BDS1445:2003,APP-B-8 | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Non detergent organic matter, % m/m | 1.00 (Max) | BDS1445:2003,APP-B-6 | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Percent detergency | 45.0 (Min) | BDS 1445:2019 | 0.00 | 14 |  | 4 |  | 0.00 |
|  |  | Hand Immersion Test | The Product shall be non-injurious to the hands of consumers and to the fabric washed with it | BDS 1445:2019 | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Biodegradability | The surfactants used in the manufacture of household laundry detergent powders shall be linear to be readily biodegradable. | BDS 1445:2019 Annex H. | 0.00 | 14 |  | 4 |  | 0.00 |
|  |  | Arsenic, mg/kg | 10.00 (Max) | AAS Method | 800.00 | 14 |  | 4 |  | 0.00 |
|  |  | Lead, mg/kg | 3.00 (Max) | AAS Method | 800.00 | 14 |  | 4 |  | 0.00 |
|  |  | Mercury, mg/kg | 1.00 (Max) | AAS Method | 800.00 | 14 |  | 4 |  | 0.00 |
|  |  | Cadmium, mg/kg | 3.00 (Max) | AAS Method | 800.00 | 14 |  | 4 |  | 0.00 |
| 26. | 26.(1) Skin Cream (BDS 1382:2024) | Description | The skin cream shall be in the form of a thick emulsion or unctuous mass, with a pleasant odour. It shall be white or pigmented or of uniform colour. | BDS 1382:2019 | 100.00 | 14 | 8000.00 | 5 | 16000.00 | 0.00 |
|  |  | Thermal stability at 45o±1ºC for 48 hours | To Pass the test | BDS 1382:2019, App-A | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH 27 °C | 5.00 to 9.00 | BDS 1382:2019, App- B | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total fatty substance content, %m/m | 5.00 (Min) | BDS 1382:2019, App- C | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Residue on evaporation 105oC, % m/m | 10.00 (Min) | BDS 1382:2019, App- D | 250.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals(as Pb), ppm | 20.00 (Max) | BDS 1382:2019, App- E | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as AS2O3), ppm | 2.00 (Max) | BDS 1333 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Microbial Content: |  |  |  | 14 |  | 5 |  | 0.00 |
|  |  | a) Total viable count cfu/gm | Not more than 1000 | BDS 1869 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Gram negative pathogens | Less than 10 | BDS 1869 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Dermatological safety (Use test) | It shall not bring harmful effect to the body. | BDS 1824. | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mercury (as Hg), ppm | 1.00 (Below) | BDS 1382:2019, App- F | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Hydroquinone, ppm | 5.00 (Below) | BDS 1382:2019, App- G | 3250.00 | 14 |  | 5 |  | 0.00 |
|  | 26.(2) Skin Cream (Only Hydroquinone) (BDS 1382:2024) | Hydroquinone, ppm | 5.00 (Below) | BDS 1382:2019, App- G | 3250.00 | 14 | 3250.00 | 3 | 3250.00 | 0.00 |
| 27. | Skin Lotion (BDS 1923:2019) | Description | The skin lotions shall be in the form of a thick emulsion or unctuous mass, with a pleasant odour. It shall be white or pigmented or of uniform color. | BDS 1923:2019 | 100.00 | 14 | 5800.00 | 5 | 11600.00 | 0.00 |
|  |  | Thermal stability at 45o±1ºC for 48 hours | Stable for two days | BDS 1923:2019 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH | 5.00 to 8.00 | BDS 1923:2019, App-A | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total fatty substance content, %m/m | 5.00 (Min) | BDS 1923:2019, App- B | 800.00 | 14 |  | 5 |  | 0.00 |
|  |  | Non Volatile Matter at 105oC, % m/m | 10.00 (Min) | BDS 1923:2019, App- C | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Water content, %m/m | 90.00 (Max) | BDS 1923:2019, App- D | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Peroxide Value, mg/Kg | 10.0 (Max) | BDS 1923:2019, App- E | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count cfu/g | Not more than 1000 | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pseudomonas aeruginosa, in 10 ml | Less than 10 | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Staphylococcus aureus, in 10 ml | Less than 10 | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
| 28. | 28(1) Shaving Cream ( Type-1) (BDS 1241:2017) | Consistency | The material shall be in the form of thick emulsion with soft texture and steady consistency .It shall be white or pigmented and of uniform colour. | BDS 1241:2017 | 100.00 | 14 | 1700.00 | 5 | 3400.00 | 0.00 |
|  |  | Homogeneity | The material shall extrude from the collapsible tube at 27 ± 2OCin the form of homogeneous mass with the application of normal force starting from the crimped end of the tube. | BDS 1241:2017 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Stability | The material shall not segregate or physically deteriorate during normal condition of storage and use. | BDS 1241:2017 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Effect of container | The material shall be packed in collapsible tubes of material, which shall not corrode or deteriorate during normal conditions of storage and use | BDS 1241:2017 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total fatty substance, %m/m | 30.00(Min) | B-2 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Water Content, %m/m | 60.00(Max) | B-3 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lathering (foaming) power, ml | 100.00(Min) | B-4 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Free caustic alkali | To pass the test | B-5 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable Count, cfu/g | 1000 (Max) | BDS 1869 | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | (a) Lead (as pb) , ppm | 20.00 (Max) | BDS 1241:2017 | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | (b) Arsenic (as AS2O3), ppm | 2.00 (Max) | BDS 1241:2017 | 300.00 | 14 |  | 5 |  | 0.00 |
|  | 28(2) Shaving Cream (Type-2) (BDS 1241:2017) | Consistency | The material shall be in the form of thick emulsion with soft texture and steady consistency .It shall be white or pigmented and of uniform colour. | BDS 1241:2017 | 100.00 | 14 | 1700.00 | 5 | 3400.00 | 0.00 |
|  |  | Homogeneity | The material shall extrude from the collapsible tube at 27 ± 2OCin the form of homogeneous mass with the application of normal force starting from the crimped end of the tube. | BDS 1241:2017 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Stability | The material shall not segregate or physically deteriorate during normal condition of storage and use. | BDS 1241:2017 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Effect of container | The material shall be packed in collapsible tubes of material, which shall not corrode or deteriorate during normal conditions of storage and use | BDS 1241:2017 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total fatty substance, %m/m | 20.00(Min) | B-2 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Water Content, %m/m | 70.00(Max) | B-3 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Free caustic alkali | To pass the test | B-5 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable Count, cfu/g | 1000 (Max) | BDS 1869 | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead (as pb) , ppm | 20.00 (Max) | BDS 1241:2017 | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as AS2O3), ppm | 2.00 (Max) | BDS 1241:2017 | 300.00 | 14 |  | 5 |  | 0.00 |
| 29. | 29 (1) After Shave Lotion (Type-1: Alcoholic) (BDS 1524:2006) | Description | The sample shall be in the form of transparent/translucent/opaque aqueous ethanolic/ emulsified solution containing perfume oil and shall be free from sediment. | BDS 1524:2006 | 100.00 | 14 | 1500.00 | 4 | 3000.00 | 0.00 |
|  |  | Alcohol Content, %V/V | 5-70 | BDS 1524:2006, ANNEX-A1.2 | 4000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Stability of smell | The smell of lotion can be clearly picked up after eight hours. . | BDS 1524:2006 CL-4.52 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Cloud temperature | No turbidity shall appear at Cooling Temp. to +5oC | BDS 1524:2006 CL-4.52 | 500.00 | 14 |  | 4 |  | 0.00 |
|  | 29(2)After Shave Lotion (Type-2: Non-Alcoholic) (BDS 1524:2006) (Type-2) | Description | The sample shall be in the form of transparent/translucent/opaque aqueous ethanolic/emulsified solution containing perfume oil and shall be free from sediment. | BDS 1524:2006 | 500.00 | 14 | 1500.00 | 4 | 3000.00 | 0.00 |
|  |  | Stability of smell | The smell of lotion can be clearly picked up after eight hours. . | BDS 1524:2006 CL-4.52 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Cloud temperature | No turbidity shall appear at Cooling Temp. to +5oC | BDS 1524:2006 CL-4.52 | 500.00 | 14 |  | 4 |  | 0.00 |
| 30. | 30 (1). Lipstick (Type-1: General Lipstick) (BDS 1424:2024) | Description | The sample shall be firm but not brittle in texture. It shall have an attractive appearance, pleasant taste and feel on the lips and shall be reasonably free from sweating bloom and rancidity. | BDS 1424:2024 | 100.00 | 14 | 3700.00 | 5 | 7400.00 | 0.00 |
|  |  | Softening point, oC | 55.00 | A-2 | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Rancidity (Peroxide value) mili equivalent Oxygen /kg | 10.00 (Max) | A-3 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Breaking load value, in gm | 200.00 | A-4 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Particle size of undispersed pigments, micron | 40.00 | A-5 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pay off test | 0.01-0.25 | A-6 | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | Free from grettiness | To pass the test | A-7 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), ppm | 2.00 (Max) | A-8 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals (as Pb), ppm | 20.00 (Max) | A-9 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mercury (as Hg), ppm | 1.00 (Max) | A-10, BDS ISO/TR 17276 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count, cfu/g | 100.00 (Max) | BDS 1869 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Gram Negative pathogens, per gram | Absent | BDS 1869 | 500.00 | 14 |  | 5 |  | 0.00 |
|  | 30(2). Lipstick (Type-2: Liquid Lipstick) (BDS 1424:2024) | Description | The sample shall be firm but not brittle in texture. It shall have an attractive appearance, pleasant taste and feel on the lips and shall be reasonably free from sweating bloom and rancidity. | BDS 1424:2024 | 100.00 | 14 | 3000.00 | 5 | 6000.00 | 0.00 |
|  |  | Rancidity (Peroxide value) mili equivalent Oxygen /kg | 10.00 (Max) | A-3 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Free from grettiness | To pass the test | A-7 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), ppm | 2.00 (Max) | A-8 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals (as Pb), ppm | 20.00 (Max) | A-9 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mercury (as Hg), ppm | 1.00 (Max) | A-10, BDS ISO/TR 17276 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count, cfu/g | 100.00 (Max) | BDS 1869 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Gram Negative pathogens, per gram | Absent | BDS 1869 | 500.00 | 14 |  | 5 |  | 0.00 |
| 31. | 31 (1). Hair Oil (Type-1 (BDS 1339: 2018) | Description | The sample shall be colourless or coloured, with or without perfumed and free from sediment suspended matter. | BDS 1339: 1991 | 100.00 | 14 | 1700.00 | 5 | 3400.00 | 0.00 |
|  |  | Acid value, mg/g | 1.00 (Max) | BDS 1339: 1991, App-A-2 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Peroxide value, milliequivalents oxygen/1000g | 7.50 (Max) | BDS 1339: 1991,App-A-3 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Microbiological Examination (Total Viable Count), CFU/g | 1000.0 (Max) | BDS 1339: 2018, App-A-3 | 500.00 | 14 |  | 5 |  | 0.00 |
|  | 31 (2). Hair Oil (Type-2) (BDS 1339: 2018) | Description | The sample shall be colourless or coloured, with or without perfumed and free from sediment suspended matter. | BDS 1339: 1991 | 100.00 | 14 | 1700.00 | 5 | 3400.00 | 0.00 |
|  |  | Acid value, mg/g | Passes the test | BDS 1339: 1991, App-A-2 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Peroxide value, milliequivalents oxygen/1000g | 7.50 (Max) | BDS 1339: 1991,App-A-3 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Microbiological Examination (Total Viable Count), CFU/g | 1000.0 (Max) | BDS 1339: 2018, App-A-3 | 500.00 | 14 |  | 5 |  | 0.00 |
| 31. | 31 (3). Hair Oil (Type-3) (BDS 1339: 2018) | Description | The sample shall be colourless or coloured, with or without perfumed and free from sediment suspended matter. | BDS 1339: 1991 | 100.00 | 14 | 1700.00 | 5 | 3400.00 | 0.00 |
|  |  | Acid value, mg/g | 1.00 (Max) | BDS 1339: 1991, App-A-2 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Peroxide value, milliequivalents oxygen/1000g | 7.50 (Max) | BDS 1339: 1991,App-A-3 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Microbiological Examination (Total Viable Count), CFU/g | 1000.0 (Max) | BDS 1339: 2018, App-A-3 | 500.00 | 14 |  | 5 |  | 0.00 |
| 32. | 32.(1) High Speed Diesel (Type-1) (BDS 344:2020) | Description | The material shall be free from grit, suspended matter and other visible impurities. | BDS 344:2020 | 100.00 | 14 | 12000.00 | 3 | 24000.00 | 0.00 |
|  |  | Density at 15oC kg/m3 | 0.820-0.860 | ASTM D 1298 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Acidity, Inorganic, mg of KOH/g | Nil | ASTM D 664 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Acidity, Total, Acid No, mg of KOH/g | 0.20 (Max) | BDS 1320,Part-24 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Ash, % m/m | 0.01 (Max) | ASTM D 482 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Carbon residue on conradson on 10 percent residue, %m/m | 0.30 (Max) | ASTM D 189 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  |  | Cetane number | 48.00 (Min) | ASTM D 613 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  |  | Cetane index | 46.00 (Min) | ASTM D 4737(A) | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Pour point,oC | 3.0 (Max) Winter | ASTM D 5950 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 9.0 (Max) Summer | ASTM D 5950 |  | 14 |  | 3 |  | 0.00 |
|  |  | Copper strip corrosion for 3 hrs at 50oC | Not worse than No-1 | ASTM D 5950, D 130 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Distillation recovery % (v/v), recovery at 365 0C | 95.0 (Min) | ASTM D 5950, D 86 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point,Pensky Martens (Close cup),oC | 55.00 (Min) | ASTM D 5950, D 93 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Water Content % v/v | 0.05(Max) | ASTM D 5950, D 95 | 600.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity in centistokes at 40ºC | 2.0-4.5 | ASTM D 5950, D 445 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Sediment, percent by mass, Max. | 0.01 (Max) | ASTM D 473 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Sulphur, parts per million (ppm), Max. | 50.00 (Max) | ASTM D 2622 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Oxidation stability | 25 20 | ASTM D 2274 ASTM D 7545 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  |  | Colour ASTM, Max. | 3.0 ( Max) | ASTM D1500 | 300.00 | 14 |  | 3 |  | 0.00 |
|  |  | Polycyclic Aromatic Hydrocarbon (PAH), Percent by mass | 8.0 ( Max) | ASTM D 6591 | 0.00 | 14 |  | 3 |  | 0.00 |
|  |  | Fatty Acid Methyl Ester (FAME) Content, % v/v | 7.0 (Max) | ASTM D 7371 | 0.00 | 14 |  | 3 |  | 0.00 |
|  | 32 (2). High Speed Diesel (Type-2) (BDS 344:2020) | Description | The material shall be free from grit, suspended matter and other visible impurities. | BDS 344:2020 | 100.00 | 14 | 12000.00 | 3 | 24000.00 | 0.00 |
|  |  | Density at 15oC kg/m3 | 0.820-0.860 | ASTM D 1298 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Acidity, Inorganic, mg of KOH/g | Nil | ASTM D 664 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Acidity,Total, Acid No, mg of KOH/g | 0.20 (Max) | ASTM D 664 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Ash, % m/m | 0.01 (Max) | ASTM D 482 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Carbon residue on conradson on 10 percent residue, % m/m | 0.30 (Max) | ASTM D 189 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  |  | Cetane number | 48.00 (Min) | ASTM D 613 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  |  | Cetane index | 46.00 (Min) | ASTM D 4737(A) | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Pour point,oC | 3.0 (Max) Winter | ASTM D 5950 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 9.0 (Max) Summer | ASTM D 5950 |  | 14 |  | 3 |  | 0.00 |
|  |  | Copper strip corrosion for 3 hrs at 50oC | Not worse than No-1 | D 5950 D 130 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Distillation recovery % (v/v), recovery at 365 0C | 95.0 (Min) | D 5950 D 86 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point,Pensky Martens (Close cup),oC | 55.00 (Min) | D 5950 D 93 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Water Content % v/v | 0.05 (Max) | D 5950 D 95 | 600.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity in centistokesat 40ºC | 2.0-4.5 | D 5950 D 445 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Sediment, percent by mass | 0.01 (Max) | ASTM D 473 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Sulphur, parts per million (ppm) | 350.00 (Max) | ASTM D 2622 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Oxidation stability a) g/m3, Max. b) Hours, Min. | a) 25(Max) b)20 (MIn) | ASTM D 2274 ASTM D 7545 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  |  | Colour ASTM, Max. | 3.0 ( Max) | ASTM D1500 | 300.00 | 14 |  | 3 |  | 0.00 |
|  |  | Polycyclic Aromatic Hydrocarbon (PAH), Percent by mass | 8.0 ( Max) | ASTM D 6591 | 0.00 | 14 |  | 3 |  | 0.00 |
|  |  | Fatty Acid Methyl Ester (FAME) Content, % v/v | 7.0 (Max) | ASTM D 7371 | 0.00 | 14 |  | 3 |  | 0.00 |
| 33. | 33. Unleaded Motor Gasoline-Premium (Octane) (BDS 346:2010, Reaffirmed 2020) | Description | The material shall be a refined petroleum distillate and free from undissolved water, foreign matter and other visible impurities. | BDS 346:2010 (Reaffirmed 2020) | 100.00 | 14 | 7800.00 | 3 | 15600.00 | 0.00 |
|  |  | Density at 15oC, Kg/L | Not limited | ASTM D 1298 | 300.00 | 14 |  | 3 |  | 0.00 |
|  |  | Colour | Red | Visual | 300.00 | 14 |  | 3 |  | 0.00 |
|  |  | Copper-strip corrosion at 50oC | Not worse than No.1 | ASTM D 130 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Distillation: |  | ASTM D 86 | 0.00 | 14 |  | 3 |  | 0.00 |
|  |  | a) Initial boiling point, oC | Not limited |  | 400.00 | 14 |  | 3 |  | 0.00 |
|  |  | b) 10% recovery by volume, oC | 75.00 (Max) |  | 400.00 | 14 |  | 3 |  | 0.00 |
|  |  | c) 50% recovery by volume, oC | 80-125 |  | 400.00 | 14 |  | 3 |  | 0.00 |
|  |  | d) 90 % recovery by volume, oC | 180.00 (Max) |  | 400.00 | 14 |  | 3 |  | 0.00 |
|  |  | e) Final boiling point, oC | 210.00 (Max) |  | 400.00 | 14 |  | 3 |  | 0.00 |
|  |  | f) Residue, %v/v | 2.00 (Max) |  | 400.00 | 14 |  | 3 |  | 0.00 |
|  |  | Octane Number | 95.0 (Min) | ASTM D 2699 | 700.00 | 14 |  | 3 |  | 0.00 |
|  |  | Lead content (as Pb), ppm | 10.0 (Max) | ASTM D 3237 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Doctor test Or Mercaptane Sulphur, Percent by mass, Max | Negative 0.001 | ASTM D 4952 ASTM D 3227 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Sulphur, parts per million (ppm), Max. | 250 (Max) | ASTM D 4294 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  |  | Oxidation stability , in minutes | 240 (Min) | ASTM D 7525 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Solvent Washed Gum Content, mg/100 ml | 4.0 (Min) | ASTM D 381 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Reid Vapour Pressure at 37.8 oC (100 oF),psig | 10 (Max) | ASTM D 5191 | 500.00 | 14 |  | 5 |  | 0.00 |
| 34. | 34.Unleaded Motor Gasoline-Regular (Petrol) (BDS 347:2019) | Description | The material shall be a refined petroleum distillate and free from undisclosed water, foreign matter and other visible impurities. | BDS: 347:2019 | 100.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 15oC kg/l | Not limited but to be reported | BDS 1320: P-1 | 300.00 | 14 |  | 3 |  | 0.00 |
|  |  | Colour, visual | Orange | Visual | 300.00 | 14 |  | 3 |  | 0.00 |
|  |  | Copper-strip corrosion for 3 hours at 50oC | Not worse than No-1 | BDS1320: P-6 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Distillation: |  | BDS1320: P-7 | 0.00 | 14 |  | 3 |  | 0.00 |
|  |  | a) Initial boiling point, oC | Not limited but to be reported |  | 400.00 | 14 |  | 3 |  | 0.00 |
|  |  | b) 10% recovery by volume, oC | 75.00 (Max) |  | 400.00 | 14 |  | 3 |  | 0.00 |
|  |  | c) 50% recovery by volume, oC | 80-125 |  | 400.00 | 14 |  | 3 |  | 0.00 |
|  |  | d) 90 % recovery by volume, oC | 180.00 (Max) |  | 400.00 | 14 |  | 3 |  | 0.00 |
|  |  | e) Final boiling point, oC | 210.00 (Max) |  | 400.00 | 14 |  | 3 |  | 0.00 |
|  |  | f) Residue, %v/v | 2.00 (Max) |  | 400.00 | 14 |  | 3 |  | 0.00 |
|  |  | Lead content (as Pb), ppm | 10.0 (Max) | BDS1320: P-10 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Doctor test Or Mercaptane Sulphur, Percent by mass, Max | Negative 0.001 | ASTM D 4952 ASTM D 3227 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Sulphur, parts per million (ppm), Max. | 300 (Max) | ASTM D 4294 | 700.00 | 14 |  | 3 |  | 0.00 |
|  |  | Oxidation stability , in minutes | 240 (Min) | ASTM D 7525 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Solvent Washed Gum Content, mg/100 ml | 4.0 (Max) | ASTM D 381 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Reid Vapour Pressure at 37.8 oC (100 oF),psig | 10 (Max) | ASTM D 5191 | 500.00 | 14 |  | 3 |  | 0.00 |
| 35 | 35.(1) Tooth Powder (Type-1, Foaming) (BDS 1370:2017) | Description | The sample shall be smooth, uniform, freeflowing fine powder. It shall be free from foreign matter. | BDS 1370:2017 | 100.00 | 14 | 2600.00 | 5 | 5200.00 | 0.00 |
|  |  | Fineness: |  | Annex-B |  | 14 |  | 5 |  | 0.00 |
|  |  | a) Particle retained on 150 micron sieve, %m/m | 1.0 (Max) |  | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Particle retained on 75 micron sieve, %m/m | 2.50 (Max) |  | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Moisture and volatile matter, %m/m | 5.00 (Max) | Annex-C | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH of 50 % aqueous suspension | 5.50-10.5 | Annex-D | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Foaming Power, in mL | 50.00 (Min) | Annex-E | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead (as Pb), ppm | 20.00 (Max) | Annex-F | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), ppm | 2.00 (Max) | Annex-G | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Abrasion | To Pass the test | Annex H/I | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count, cfu/g | 1000 (Max) | BDS 1869:2013 | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Gram negative Pathogens/g | Absent | BDS 1869:2013 | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Fluoride ion concentration, ppm | 1000.00 (Max) | BDS 1216:2012, App-I | 0.00 | 14 |  | 5 |  | 0.00 |
|  | 35(2). Tooth Powder (Type-2,Non-Foaming) (BDS 1370:2017) | Description | The sample shall be smooth, uniform, freeflowing fine powder. It shall be free from foreign matter. | BDS 1370:2017 | 100.00 | 14 | 2600.00 | 5 | 5200.00 | 0.00 |
|  |  | Fineness: |  | Annex-B |  | 14 |  | 5 |  | 0.00 |
|  |  | a) Particle retained on 150 micron sieve, %m/m | 1.0 (Max) |  | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Particle retained on 75 micron sieve, %m/m | 2.50 (Max) |  | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Moisture and volatile matter, %m/m | 5.00 (Max) | Annex-C | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH of 50 % aqueous suspension | 5.50-10.5 | Annex-D | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Foaming Power, in mL | N/A | Annex-E | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead (as Pb), ppm | 20.00 (Max) | Annex-F | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), ppm | 2.00 (Max) | Annex-G | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Abrasion | To Pass the test | Annex H/I | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count, cfu/g | 1000 (Max) | BDS 1869:2013 | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Gram negative Pathogens/g | Absent | BDS 1869:2013 | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Fluoride ion concentration, ppm | 1000.00 (Max) | BDS 1216:2012, App-I | 0.00 | 14 |  | 5 |  | 0.00 |
| 36. | 36 a). Skin Powder (Face Powder) (BDS 1337:2024) | Description | The face powder shall consist principally of a finely powdered free flowing absorbent innocuous material such as natural talc and may contain small amounts of perfume and colouring matter. | BDS 1337:2024 | 100.00 | 14 | 2600.00 | 5 | 5200.00 | 0.00 |
|  |  | Matter insoluble in boiling water, %m/m | 90.00 (Min) | BDS 1337:2024, App-B | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Fineness: |  |  | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | a)Residue on 75 micron sieve, %m/m | 1.00 (Max) | BDS 1337:2024, App-C | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Residue on 150 micron sieve, %m/m | 0.50 (Max) | BDS 1337:2024, App-C | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Moisture and volatile matter, % m/m | 3.00 (Max) | BDS 1337:2024, App-D | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH of aqueous suspension | 5.5 to 9.00 | BDS 1337:2024, App-E | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metal (as Pb), ppm | 20.00 (Max) | BDS 1337:2024, App-F | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as AS2O3), ppm | 2.00 (Max) | BDS 1337:2024, App-G | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mercury (Hg), ppm | 1.00 (Max) | BDS ISO/TR 17276 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Boric acid, borates and tetra borates; percent by mass (as boric acid); Max | 5.00 (Max) | BDS 1337:2024, App-I | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Dermatological safety (use test) | It Shall not bring harmful effect to the body. | BDS 1337:2024 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | a) Total viable count cfu/g, Max | 1000 | BDS 1869 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Yeast and mould count, CFU/g, Max | 100 | BDS ISO 16212 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | c) Pseudomonas aeruginosa, per gram | Absent | BDS ISO 22717 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | d) Escherichia coli, per gram | Absent | BDS ISO 21150 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | e) Staphylococcus aureus, per gram | Absent | BDS ISO 22718 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | f) Candida albicans, per gram | Absent | BDS ISO 18416 | 200.00 | 14 |  | 5 |  | 0.00 |
|  | 36 (2). Skin Powder (Body powder, Regular) (BDS 1337:2024) | Description | The face powder shall consist principally of a finely powdered free flowing absorbent innocuous material such as natural talc and may contain small amounts of perfume and colouring matter. | BDS 1337:2024 | 100.00 | 14 | 2600.00 | 5 | 5200.00 | 0.00 |
|  |  | Matter insoluble in boiling water, %m/m | 90.00 (Min) | BDS 1337:2024, App-B | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Fineness: |  |  | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | a)Residue on 75 micron sieve, %m/m | 2.00 (Max) | BDS 1337:2024, App-C | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Residue on 150 micron sieve, %m/m | 0.50 (Max) | BDS 1337:2024, App-C | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Moisture and volatile matter, % m/m | 2.00 (Max) | BDS 1337:2024, App-D | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH of aqueous suspension | 5.5 to 10.00 | BDS 1337:2024, App-E | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metal (as Pb), ppm | 20.00 (Max) | BDS 1337:2024, App-F | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as AS2O3), ppm | 2.00 (Max) | BDS 1337:2024, App-G | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mercury (Hg), ppm | 1.00 (Max) | BDS ISO/TR 17276 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Boric acid, borates and tetra borates; percent by mass (as boric acid); Max | 5.00 (Max) | BDS 1337:2024, App-I | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Dermatological safety (use test) | It Shall not bring harmful effect to the body. | BDS 1337:2024 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count cfu/g, Max | 1000 | BDS 1869 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Yeast and mould count, CFU/g, Max | 100 | BDS ISO 16212 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pseudomonas aeruginosa, per gram | Absent | BDS ISO 22717 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Escherichia coli, per gram | Absent | BDS ISO 21150 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Staphylococcus aureus, per gram | Absent | BDS ISO 22718 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Candida albicans, per gram | Absent | BDS ISO 18416 | 200.00 | 14 |  | 5 |  | 0.00 |
|  | 36(3) Skin Powder (Body powder, Cool) (BDS 1337:2024) | Description | The face powder shall consist principally of a finely powdered free flowing absorbent innocuous material such as natural talc and may contain small amounts of perfume and colouring matter. | BDS 1337:2024 | 100.00 | 14 | 2600.00 | 5 | 5200.00 | 0.00 |
|  |  | Matter insoluble in boiling water, %m/m | 90.00 (Min) | BDS 1337:2024, App-B | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Fineness: |  |  | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | a)Residue on 75 micron sieve, %m/m | 2.00 (Max) | BDS 1337:2024, App-C | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Residue on 150 micron sieve, %m/m | 0.50 (Max) | BDS 1337:2024, App-C | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Moisture and volatile matter, % m/m | 5.00 (Max) | BDS 1337:2024, App-D | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH of aqueous suspension | 5.5 to 10.00 | BDS 1337:2024, App-E | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metal (as Pb), ppm | 20.00 (Max) | BDS 1337:2024, App-F | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as AS2O3), ppm | 2.00 (Max) | BDS 1337:2024, App-G | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mercury (Hg), ppm | 1.00 (Max) | BDS ISO/TR 17276 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Boric acid, borates and tetra borates; percent by mass (as boric acid); Max | 5.00 (Max) | BDS 1337:2024, App-I | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Dermatological safety (use test) | It Shall not bring harmful effect to the body. | BDS 1337:2024 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Menthol content, percent by mass, Max | 1.00 (Max) | BDS 1337:2024, App-J | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | Camphor content, percent by mass, Max | 1.50 (Max) | BDS 1337:2024, App-J | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count cfu/g, Max | 1000 | BDS 1869 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Yeast and mould count, CFU/g, Max | 100 | BDS ISO 16212 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pseudomonas aeruginosa, per gram | Absent | BDS ISO 22717 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Escherichia coli, per gram | Absent | BDS ISO 21150 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Staphylococcus aureus, per gram | Absent | BDS ISO 22718 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Candida albicans, per gram | Absent | BDS ISO 18416 | 200.00 | 14 |  | 5 |  | 0.00 |
| 37. | 37.M S Tube (G I Pipe | Weight of zinc coating, gm/m2 |  |  | 600 | 14 |  | 5 |  | 0.00 |
|  |  | Phosphorous, %m/m |  |  | 600 | 14 |  | 5 |  | 0.00 |
|  |  | Sulphur (as S), %m/m |  |  | 600 | 14 |  | 5 |  | 0.00 |
|  |  | Carbon(as C), %m/m |  |  | 600 | 14 |  | 5 |  | 0.00 |
|  |  | Manganese(as Mn), %m/m |  |  | 600 | 14 |  | 5 |  | 0.00 |
|  | 38.GP sheet (Corrugated Iron Sheet) (BDS 1122:1987) | i) Thickness with Zn-coating, mm | 0.394 (Min) |  | 500.00 | 14 | 2000.00 | 4 | 4000.00 | 0.00 |
|  |  | ii) Thickness of base metal, mm | 0.350 (Min) |  | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | iii) Thickness of Zn-coating, mm | 0.044 (Min) |  | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Weight of zinc coating (Z 22), g/m2 |  |  | 500.00 | 14 |  | 4 |  | 0.00 |
|  | 39. Bitumen Road emulsion |  |  |  |  | 14 | 1500.00 | 4 | 3000.00 | 0.00 |
| 40. | Paving Grade Bitumen (Grade 50/70) (BDS EN12591:2009) | Penetration at 25 ºC, (100g, 5 Sec) x 0.1 mm | 50 - 70 | EN 1426 | 2000.00 | 14 | 8000.00 | 4 | 16000.00 | 0.00 |
|  |  | Softening Point (Ring & Ball) ºC | 46 - 54 | EN 1427 | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Flash point (Cleveland Open Cup) ºC | 230 (Min) | EN 22592 (b) | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Solubility, (in xylene) % m/m | 99.00 (Min) | EN 12592 | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Change of Mass, % | 0.50 (Max) | EN 12607-1 or EN 12607-3 or | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Retained penetration, % | 50.00 (min) | EN 12607-1 or EN 12607-3 or | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Softening Point (After hardening), 0C | 48 (Min) | EN 1427 | 1000.00 | 14 |  | 4 |  | 0.00 |
| 41 | 41(1)Framework Specification for Polymer Modified Bitumen (Class-5) (BDS EN 14023:2009) | Consistency at Intermediate Service Temperature, Penetration at 25 ºC, (100g, 5 Sec) x 0.1 mm | 40-100 | EN 1428 | 1500.00 | 14 | 7000.00 | 4 | 14000.00 | 0.00 |
|  |  | Consistency at Elevated Service Temperature, Softening Point (Ring & Ball) ºC | ≤65 | EN 1427 | 1500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Durability (Resistance to hardening, Change of Mass, % | ≤ 1.00 | EN 12607-1 or EN 12607-3 or | 2000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Durability (Resistance to hardening, Retained penetration, % m/m | ≥ 50.00 | EN 12607-1 or EN 12607-3 or | 2000.00 | 14 |  | 4 |  | 0.00 |
|  | 41(2)Framework Specification for Polymer Modified Bitumen (Class-4) (BDS EN 14023:2009) | Consistency at Intermediate Service Temperature, Penetration at 25 ºC, (100g, 5 Sec) x 0.1 mm | 45 - 80 | EN 1426 | 1500.00 | 14 | 7000.00 | 4 | 14000.00 | 0.00 |
|  |  | Consistency at Elevated Service Temperature, Softening Point (Ring & Ball) ºC | ≤ 70 | EN 1427 | 1500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Durability (Resistance to hardening) Change of Mass, % | ≤ 0.8 | EN 12607-1 or EN 12607-3 or | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Durability (Resistance to hardening) Retained penetration, % | ≥ 45 | EN 12607-1 or EN 12607-3 or | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Durability (Resistance to hardening) Increase in Softening Point, 0C | ≤ 12.00 | EN 1427 | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Other Characteristics, Flash Point, 0C | ≥ 220 | EN ISO 2592 | 1000.00 | 14 |  | 4 |  | 0.00 |
| 42. | Steel for the Reinforcement of concrete, Ribbed Bar (BDS ISO 6935-2 :2021) | Carbon (C), % m/m | 0.32 (Max) | IS 228(Part-1): 1987 | 1000.00 | 14 | 5000.00 | 10 | 10000.00 | 0.00 |
|  |  | Silicon (Si), %m/m | 0.55 (Max) | ASTM-350:2012, Section-46. | 1000.00 | 14 |  | 10 |  | 0.00 |
|  |  | Manganese (Mn), %m/m | 1.80 (Max) | ASTM-350:2012, Section-164. | 1000.00 | 14 |  | 10 |  | 0.00 |
|  |  | Phosphorous (P), % m/m | 0.040 (Max) | ASTM-350:2012, Alkali metric method Section-172. | 1000.00 | 14 |  | 10 |  | 0.00 |
|  |  | Sulphur (S), % m/m | 0.040 (Max) | IS 228(Part-9): 1989, Evolution method, | 1000.00 | 14 |  | 10 |  | 0.00 |
| 43. | 43(1) Portland Composite Cement (Type-CEM II (Strength Class: 32.5 N, 32.5 R, 42.5 N)) (BDS EN 197-1:2023) | Sulfate content (as SO3), % m/m | ≤ 3.5 | BDS EN 196-2 | 1000.00 | 14 | 2000.00 | 3 | 4000.00 | 0.00 |
|  |  | Chloride content (as Cl), % m/m | ≤ 0.10 | BDS EN 196-2 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 43(2) Portland Composite Cement (Type-CEM II (Strength Class: 42.5 R, 52.5 N, 52.5 R) (BDS EN 197-1:2023) | Sulfate content (as SO3), % m/m | ≤ 4.0 | BDS EN 196-2 | 1000.00 | 14 | 2000.00 | 3 | 4000.00 | 0.00 |
|  |  | Chloride content (as Cl), % m/m | ≤ 0.10 | BDS EN 196-2 | 1000.00 | 14 |  | 3 |  | 0.00 |
| 44. | 44(1) Portland Cement (Type-CEM I (Strength Class: 32.5 N, 32.5 R, 42.5 N) (BDS EN 197-1:2023) | Loss on ignition, %m/m | ≤ 5.0 | BDS EN 196-2b | 1200.00 | 14 | 4500.00 | 3 | 9000.00 | 0.00 |
|  |  | Insoluble residue, % m/m | ≤ 5.0 | BDS EN 196-2 | 1300.00 | 14 |  | 3 |  | 0.00 |
|  |  | Sulfate content (as SO3), % m/m (Strength | ≤ 3.5 | BDS EN 196-2 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  |  | Chloride content (as Cl), % m/m | ≤ 0.10 | BDS EN 196-2 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 44(2)Portland Cement (Type- CEM I (Strength Class: 42.5 R, 52.5 N, 52.5 R) (BDS EN 197-1:2023) | Loss on ignition, %m/m | ≤ 5.0 | BDS EN 196-2b | 1200.00 | 14 | 4500.00 | 3 | 9000.00 | 0.00 |
|  |  | Insoluble residue, % m/m | ≤ 5.0 | BDS EN 196-2 | 1300.00 | 14 |  | 3 |  | 0.00 |
|  |  | Sulfate content (as SO3), % m/m (Strength | ≤ 4.0 | BDS EN 196-2 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  |  | Chloride content (as Cl), % m/m | ≤ 0.10 | BDS EN 196-2 | 1000.00 | 14 |  | 3 |  | 0.00 |
| 45. | U-PVC Pipe (BDS 1878:2015) | Appearance | Visually the internal and external surface of the pipe shall be smooth, clean and free from scoring cavities and surface defects. The material shall not contain visible impurities. | BDS 1878:2015 | 100.00 | 14 | 2300.00 | 5 | 4600.00 | 0.00 |
|  |  | Immersion test in Acetone (Alternative test method. In case of dispute resistance to dichloromethane shall be used) | The material shall not show any sign of delimitation or disintegration | BDS 1878:2015, Annex C | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Effect of materials on water quality a) Lead (Pb) (First extraction) mg/l | 1.0 mg/l (1.0 part per million by mass) | BDS 1878:2015, Annex - D | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Lead (Pb) (Third extraction) mg/l | 0.3 mg/l (0.3 part per million by mass) | BDS 1878:2015, Annex - D | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | c) Dialkyl tin C4 and higher homologues measured as tin (Third extraction) mg/l | 0.2 mg/l (0.2 part per million by mass) | BDS 1878:2015, Annex - D | 600.00 | 14 |  | 5 |  | 0.00 |
| 46. | Sanitary Ware Appliances (Commode/Basin) (BDS 1162:2014) | Description | The material shall not show to the unaided eye of a trained observer, blemishes or defects of discoloration, spot, blisters and pinholes, bubbles and specks. | BDS 1162:2006 | 100.00 | 14 | 2000.00 | 4 | 4000.00 | 0.00 |
|  |  | Resistance to Chemicals | The material shall not show any loss of reflectivity on the glaze of the sample. | BDS 1162:2006, App-C | 800.00 | 14 |  | 4 |  | 0.00 |
|  |  | Resistance to staining and burning. | No stain shall remain on the test pieces. | BDS 1162:2006, App-D | 1100.00 | 14 |  | 4 |  | 0.00 |
|  | 47.Ceramic Tableware of Melamine Plastics (BDS 485:2000,Amend-2006 ) | Material | The table ware shall be fired ceramic ware covered by glaze properly matured and fused with the body. The body shall show upon fracture a dense homogeneous vitrified texture. The glaze shall be of uniform colour, impervious and as free as possible from visual defects. | BDS 485:2000 Cl-4.1 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Finish | Except for the resting surface (bottom rim) of the hollow ware, the entire surface of items of table ware shall be covered by a uniform and continuous, hard impervious glaze, which is cleanable and retain this quality in addition the resting surface of items other than hollow ware shall be non-abrasive in texture. The glazed surface shall be free from defects namely craze, Crawling patches and finger prints. | BDS 485:2000, Cl-4.3 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Wet ability | Table ware used for eating purposes shall pass the test. | BDS 485:2000, Cl-5.3 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cadmium (Cd). released, mg/L | 0.25 (Max) | ISO 6486-1:1999, Cl-4.0 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead (Pb), released, mg/L | 0.5 (Max) | ISO 6486-1:1999, Cl-4.0 | 1000.00 | 14 |  | 5 |  | 0.00 |
| 48. | Ceramic Tiles (BDS ISO 13006:2015) | Resistance to staining | Class -3 (Min) | ISO 10545-14 | 900.00 | 14 | 1800.00 | 6 | 3600.00 | 0.00 |
|  |  | Resistance to chemical: |  |  | 0.00 | 14 |  | 6 |  | 0.00 |
|  |  | a) Resistance to low concentrations of acids & alkalis | There shall be no attack on the surface. | ISO 10545-13 | 300.00 | 14 |  | 6 |  | 0.00 |
|  |  | b) Resistance to high concentrations of acids & alkalis | There shall be no attack on the surface. | ISO 10545-13 | 300.00 | 14 |  | 6 |  | 0.00 |
|  |  | c) Resistance to household chemicals and swimming pool salts | Minimum GB | ISO 10545-13 | 300.00 | 14 |  | 6 |  | 0.00 |
| 49 | 49(1)Mosquito Coil (Active Ingredient Content (D-allethrin)) (BDS 1089:2018) | Description | The sample shall be in twin form, and it will be easily separated without breaking. | BDS: 1089:2018 | 100.00 | 14 | 6100.00 | 4 | 7200.00 | 0.00 |
|  |  | Dropping test | The coil shall remain intact after being dropped from a height of 1.5 meters with the plane of the coil horizontal to a smooth concrete face. | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Burning characteristic | 6 hours (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Length | 70.0 cm (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Mass Per Pair of Coils | 24.0 g (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Strength | The sample shall be able to withstand a minimum load of 120g | BDS: 1089:2018, Annex – F. | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Separation of Joint Coils | When 50 twin coils are separated the number of twin coils that are broken shall not be more than 3 | BDS: 1089:2018, Annex – G | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Moisture, % m/m | 12.0 (Max) | BDS: 1089:2018, Appx-H | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Active Ingredient Content (D-allethrin), Percent by mass | 0.1 – 0.3 | Annex C of BDS: 1089:2018/GLC | 5000.00 | 14 |  | 4 |  | 0.00 |
|  | 49(2) Mosquito Coil (Active Ingredient Content(D-Trans-allethrin))) (BDS 1089:2018) | Description | The sample shall be in twin form, and it will be easily separated without breaking. | BDS: 1089:2018 | 100.00 | 14 | 6100.00 | 4 | 7200.00 | 0.00 |
|  |  | Dropping test | The coil shall remain intact after being dropped from a height of 1.5 meters with the plane of the coil horizontal to a smooth concrete face. | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Burning characteristic | 6 hours (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Length | 70.0 cm (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Mass Per Pair of Coils | 24.0 g (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Strength | The sample shall be able to withstand a minimum load of 120g | BDS: 1089:2018, Annex – F. | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Separation of Joint Coils | When 50 twin coils are separated the number of twin coils that are broken shall not be more than 3 | BDS: 1089:2018, Annex – G | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Moisture, % m/m | 12.0 (Max) | BDS: 1089:2018, Appx-H | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Active Ingredient Content (D-Trans-allethrin), Percent by mass | 0.05 – 0.3 | Annex D of BDS: 1089:2018/GLC | 5000.00 | 14 |  | 4 |  | 0.00 |
|  | 49(3) Mosquito Coil (Active Ingredient Content (Dimefluthrin) (BDS 1089:2018) | Description | The sample shall be in twin form, and it will be easily separated without breaking. | BDS: 1089:2018 | 100.00 | 14 | 6100.00 | 4 | 7200.00 | 0.00 |
|  |  | Dropping test | The coil shall remain intact after being dropped from a height of 1.5 meters with the plane of the coil horizontal to a smooth concrete face. | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Burning characteristic | 6 hours (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Length | 70.0 cm (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Mass Per Pair of Coils | 24.0 g (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Strength | The sample shall be able to withstand a minimum load of 120g | BDS: 1089:2018, Annex – F. | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Separation of Joint Coils | When 50 twin coils are separated the number of twin coils that are broken shall not be more than 3 | BDS: 1089:2018, Annex – G | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Moisture, % m/m | 12.0 (Max) | BDS: 1089:2018, Appx-H | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Active Ingredient Content (Dimefluthrin), Percent by mass | 0.004 – 0.03 | GLC | 5000.00 | 14 |  | 4 |  | 0.00 |
|  | 49(4) Mosquito Coil (Active Ingredient Content (Metofluthrin) (BDS 1089:2018) | Description | The sample shall be in twin form, and it will be easily separated without breaking. | BDS: 1089:2018 | 100.00 | 14 | 6100.00 | 4 | 7200.00 | 0.00 |
|  |  | Dropping test | The coil shall remain intact after being dropped from a height of 1.5 meters with the plane of the coil horizontal to a smooth concrete face. | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Burning characteristic | 6 hours (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Length | 70.0 cm (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Mass Per Pair of Coils | 24.0 g (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Strength | The sample shall be able to withstand a minimum load of 120g | BDS: 1089:2018, Annex – F. | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Separation of Joint Coils | When 50 twin coils are separated the number of twin coils that are broken shall not be more than 3 | BDS: 1089:2018, Annex – G | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Moisture, % m/m | 12.0 (Max) | BDS: 1089:2018, Appx-H | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Active Ingredient Content (Metofluthrin), Percent by mass | 0.004 – 0.03 | GLC | 5000.00 | 14 |  | 4 |  | 0.00 |
|  | 49(5) Mosquito Coil (Active Ingredient Content (Prallethrin) (BDS 1089:2018) | Description | The sample shall be in twin form, and it will be easily separated without breaking. | BDS: 1089:2018 | 100.00 | 14 | 6100.00 | 4 | 7200.00 | 0.00 |
|  |  | Dropping test | The coil shall remain intact after being dropped from a height of 1.5 meters with the plane of the coil horizontal to a smooth concrete face. | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Burning characteristic | 6 hours (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Length | 70.0 cm (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Mass Per Pair of Coils | 24.0 g (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Strength | The sample shall be able to withstand a minimum load of 120g | BDS: 1089:2018, Annex – F. | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Separation of Joint Coils | When 50 twin coils are separated the number of twin coils that are broken shall not be more than 3 | BDS: 1089:2018, Annex – G | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Moisture, % m/m | 12.0 (Max) | BDS: 1089:2018, Appx-H | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Active Ingredient Content (Prallethrin), Percent by mass | 0.03 – 0.08 | GLC | 5000.00 | 14 |  | 4 |  | 0.00 |
|  | 49(6) Mosquito Coil (Active Ingredient Content (Trans-fluthrin) (BDS 1089:2018) | Description | The sample shall be in twin form, and it will be easily separated without breaking. | BDS: 1089:2018 | 100.00 | 14 | 6100.00 | 4 | 7200.00 | 0.00 |
|  |  | Dropping test | The coil shall remain intact after being dropped from a height of 1.5 meters with the plane of the coil horizontal to a smooth concrete face. | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Burning characteristic | 6 hours (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Length | 70.0 cm (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Mass Per Pair of Coils | 24.0 g (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Strength | The sample shall be able to withstand a minimum load of 120g | BDS: 1089:2018, Annex – F. | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Separation of Joint Coils | When 50 twin coils are separated the number of twin coils that are broken shall not be more than 3 | BDS: 1089:2018, Annex – G | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Moisture, % m/m | 12.0 (Max) | BDS: 1089:2018, Appx-H | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Active Ingredient Content (Trans-fluthrin), Percent by mass | 0.02 – 0.05 | GLC | 5000.00 | 14 |  | 4 |  | 0.00 |
|  | 49(7) Mosquito Coil (Active Ingredient Content (Bifenthrin) (BDS 1089:2018) | Description | The sample shall be in twin form, and it will be easily separated without breaking. | BDS: 1089:2018 | 100.00 | 14 | 6100.00 | 4 | 7200.00 | 0.00 |
|  |  | Dropping test | The coil shall remain intact after being dropped from a height of 1.5 meters with the plane of the coil horizontal to a smooth concrete face. | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Burning characteristic | 6 hours (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Length | 70.0 cm (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Mass Per Pair of Coils | 24.0 g (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Strength | The sample shall be able to withstand a minimum load of 120g | BDS: 1089:2018, Annex – F. | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Separation of Joint Coils | When 50 twin coils are separated the number of twin coils that are broken shall not be more than 3 | BDS: 1089:2018, Annex – G | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Moisture, % m/m | 12.0 (Max) | BDS: 1089:2018, Appx-H | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Active Ingredient Content (Bifenthrin), Percent by mass | 00.05 -0.1 | GLC | 5000.00 | 14 |  | 4 |  | 0.00 |
|  | 49(8) Mosquito Coil (Active Ingredient Content (Meperfluthrin) (BDS 1089:2018 | Description | The sample shall be in twin form, and it will be easily separated without breaking. | BDS: 1089:2018 | 100.00 | 14 | 6100.00 | 4 | 7200.00 | 0.00 |
|  |  | Dropping test | The coil shall remain intact after being dropped from a height of 1.5 meters with the plane of the coil horizontal to a smooth concrete face. | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Burning characteristic | 6 hours (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Length | 70.0 cm (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Mass Per Pair of Coils | 24.0 g (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Strength | The sample shall be able to withstand a minimum load of 120g | BDS: 1089:2018, Annex – F. | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Separation of Joint Coils | When 50 twin coils are separated the number of twin coils that are broken shall not be more than 3 | BDS: 1089:2018, Annex – G | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Moisture, % m/m | 12.0 (Max) | BDS: 1089:2018, Appx-H | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Active Ingredient Content (Meperfluthrin), Percent by mass | 0.05 – 0.08 | GLC | 5000.00 | 14 |  | 4 |  | 0.00 |
|  | 49(9) Mosquito Coil (Active Ingredient Content (Esbiothrin) (BDS 1089:2018 | Description | The sample shall be in twin form, and it will be easily separated without breaking. | BDS: 1089:2018 | 100.00 | 14 | 6100.00 | 4 | 7200.00 | 0.00 |
|  |  | Dropping test | The coil shall remain intact after being dropped from a height of 1.5 meters with the plane of the coil horizontal to a smooth concrete face. | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Burning characteristic | 6 hours (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Length | 70.0 cm (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Mass Per Pair of Coils | 24.0 g (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Strength | The sample shall be able to withstand a minimum load of 120g | BDS: 1089:2018, Annex – F. | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Separation of Joint Coils | When 50 twin coils are separated the number of twin coils that are broken shall not be more than 3 | BDS: 1089:2018, Annex – G | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Moisture, % m/m | 12.0 (Max) | BDS: 1089:2018, Appx-H | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Active Ingredient Content (Esbiothrin), Percent by mass | 0.05 – 0.3 | GLC | 5000.00 | 14 |  | 4 |  | 0.00 |
|  | 49(10) Mosquito Coil (Active Ingredient Content (S - bioallethrin) (BDS 1089:2018 | Description | The sample shall be in twin form, and it will be easily separated without breaking. | BDS: 1089:2018 | 100.00 | 14 | 6100.00 | 4 | 7200.00 | 0.00 |
|  |  | Dropping test | The coil shall remain intact after being dropped from a height of 1.5 meters with the plane of the coil horizontal to a smooth concrete face. | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Burning characteristic | 6 hours (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Length | 70.0 cm (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Mass Per Pair of Coils | 24.0 g (Min) | BDS: 1089:2018 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Strength | The sample shall be able to withstand a minimum load of 120g | BDS: 1089:2018, Annex – F. | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Separation of Joint Coils | When 50 twin coils are separated the number of twin coils that are broken shall not be more than 3 | BDS: 1089:2018, Annex – G | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Moisture, % m/m | 12.0 (Max) | BDS: 1089:2018, Appx-H | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Active Ingredient Content (S - bioallethrin), Percent by mass | 0.05 – 0.3 | GLC | 5000.00 | 14 |  | 4 |  | 0.00 |
| 50. | 50.Melathion 57% (w/v): ---- |  |  |  |  | 14 | 3200.00 | 3 | 6400.00 | 0.00 |
| 51. | Baby Toilet Soap (BDS 1798:2019) | Total fatty matter, %m/m | 78.00 (Min) | Annex B | 600.00 | 14 | 5500.00 | 4 | 11000.00 | 0.00 |
|  |  | Moisture and Volatile Matter, %m/m | 15.0 (Max) | BDS ISO 672 | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Matter insoluble in ethanol, % m/m | 1.50 (Max) | BDS ISO 673 | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Matter insoluble in water, % m/m | 0.50 (Max) | Annex C | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Free caustic alkali (as NaOH), % m/m | 0.03 (Max) | BDS ISO 456 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Chlorides (as NaCl), % m/m | 1.00 (Max) | BDS ISO 457 | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Free Carbonated alkali, % m/m | 0.50 (Max) | Annex D | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Free from Rosin | To pass the test | Annex E | 400.00 | 14 |  | 4 |  | 0.00 |
|  |  | Free from Grit | To pass the test | Annex F | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Nickel content | Nil | Annex G | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Iron content, ppm | 10.00 (Max) | Annex H | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Copper (as Cu) contents, ppm | 3.00 (Max) | Annex I | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Arsenic (as As2O3), mg/Kg, | 1.50 (Max) | BDS ISO/TR 17276 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Lead (as Pb), mg/Kg, | 10.00 (Max) | BDS ISO/TR 17276 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Mercury (as Hg), mg/Kg, | 1.00 (Max) | BDS ISO/TR 17276 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Titre,°C | 25.00 (Min) | BDS 406, Clause-7 | 500.00 | 14 |  | 4 |  | 0.00 |
| 52. | 52.Diammonium Phosphate (BDS 1628: 2000) | Description | The material shall be in the form of free-flowing granules. | BDS: 1628:2000; Cl No. 2.1 | 100.00 | 14 | 3750.00 | 3 | 7500.00 | 0.00 |
|  |  | Total Nitrogen (in ammoniacal form) percent by mass, min | 18.00 | Appendix A-4, BDS 36:1993 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total phosphates (as P2O5) percent by mass, min | 46.00 | Appendix C, BDS 67:1998 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  |  | Water soluble phosphates (as P2O5) percent by mass, min | 41.00 | Appendix D, BDS 68:1998 | 650.00 | 14 |  | 3 |  | 0.00 |
|  |  | Moisture, percent by mass, Max | 1.00 | Appendix A, BDS 68:1998 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Particle size (sieve) | Minimum 90% of the material shall pass through 4 mm BDS sieve and be retained on 1 mm BDS sieve. | BDS: 1628:2000 | 250.00 | 14 |  | 3 |  | 0.00 |
|  |  | Particle size | No more than 5% shall be lower than 1 mm size | BDS: 1628:2000 | 250.00 | 14 |  | 3 |  | 0.00 |
| 53. | Facial Tissue Paper (BDS 1723:2022) | Description | The sample shall be creped, highly absorbent and soft. It shall be free from embedded abrasive particles and other visible defects. It shall be minimum two-ply. | BDS 1723:2022 | 100.00 | 14 | 2000.00 | 5 | 4000.00 | 0.00 |
|  |  | pH (at room temperature) | 6.00 – 8.50 | BDS ISO 6588-1 | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Elementary Chlorine (as Cl) | Absent |  | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | a) Total plate count, cfu/g, Max. | Not detectable in 1 g of the product | BDS ISO 8784-1 & ISO 8784-2 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pseudomonas aeruginosa, cfu/g | Not detectable in 1 g of the product | ISO 22717 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Staphylococcus aureus, cfu/g | Not detectable in 1 g of the product | ISO 22718 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Candida albicans, cfu/g | Not detectable in 1 g of the product | ISO 18416 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Escherichia coli, cfu/g | Not detectable in 1 g of the product | ISO 21150 | 200.00 | 14 |  | 5 |  | 0.00 |
| 54. | Toilet Tissue Paper (BDS 1745:2022) | Description | The paper shall clean and free from foreign matter, stains and holes. In a roll, it shall be possible to separate the sheets cleanly at the perforations. | BDS 1745:2022 | 50.00 | 14 | 1250.00 | 5 | 2500.00 | 0.00 |
|  |  | pH (at room temperature) | 6.0 – 8.5 | BDS 1745:2022 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total Plate count,cfu/g Max | 300 | BDS ISO 8784-1 ISO 8784-2 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pseudomonas aeruginosa,,cfu/g | Not detectable in 1g of the product | ISO 227147 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Staphylococcus aureus, cfu/g | Not detectable in 1g of the product | ISO 22718 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Candida albicans, cfu/g | Not detectable in 1g of the product | ISO 18416 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Escherichia coli, cfu/g | Not detectable in 1g of the product | ISO 21150 | 200.00 | 14 |  | 5 |  | 0.00 |
| 55. | Sanitary Napkin (BDS 1261: 2019) | Total Viable Bacterial Count per Sanitary napkin | 1000 (Max) | BDS 1261:2019 | 1000.00 | 14 | 1000.00 | 5 | 2000.00 | 0.00 |
| 56. | 56 (1) Baby Oil (Type-1) (BDS 1766:2019) | Description | The sample shall be clear, colorless light mineral oil or vegetable oil containing emollients. It may be perfumed and free from foreign matter. | BDS 1766:2019 | 100.00 | 14 | 6400.00 | 5 | 12800.00 | 0.00 |
|  |  | Peroxide Value, milliequivalent/kg | 0.6 (Max) | BDS 1420 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Relative Density at 20 O C | 0.82-0.88 | BDS 1584 :Pt-1 | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Saponication Value | 4.8 (Max) | BDS 1584 :Pt-1 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Acid Value | 0.14 (Max) | BDS 1584 :Pt-1 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Sulfur and Sulfides | To pass the test | Annex B | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic, mg/kg | 1.00 (Max) | 15 of IS 1699 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead (pb), mg/kg | 2.00 (Max) | 15 of IS 1699 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cadmium, mg/kg, max. | - | 15 of IS 1699 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mercury, mg/kg, max. | - | 15 of IS 1699 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Aerobic Plate Count, per ml, | 500 (Max) | BDS 1869 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pseudomonas aeruginosa | Absent in 10 ml | BDS 1869 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Staphylococcus aureus | Absent in 10 ml | BDS 1869 | 500.00 | 14 |  | 5 |  | 0.00 |
|  | 56.(2) Baby Oil (BDS 1766:2019) (Type-2,vegetable oil) | Description | The sample shall be clear, colorless light mineral oil or vegetable oil containing emollients. It may be perfumed and free from foreign matter. | BDS 1766:2019 | 100.00 | 14 | 6400.00 | 5 | 12800.00 | 0.00 |
|  |  | Peroxide Value, milliequivalent/kg | 8.00 (Max) | BDS 1420 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Relative Density at 200C | 0.82-0.96 | BDS 1584 : Pt-1 | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Saponication Value | 250.00 (Max) | BDS 1584 : Pt-1 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Acid Value | 0.80 (Max) | BDS 1584 : Pt-1 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Sulfur and Sulfides | To pass the test | Annex B | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic, mg/kg | 1.00 (Max) | 15 of IS 1699 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead (pb), mg/kg | 2.00 (Max) | 15 of IS 1699 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cadmium, mg/kg, max. | 1.00 | 15 of IS 1699 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mercury, mg/kg, max. | 1.00 | 15 of IS 1699 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | i) Aerobic Plate Count, per ml, | 500 (Max) | BDS 1869 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | ii) Pseudomonas aeruginosa | Absent in 10 ml | BDS 1869 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | iii) Staphylococcus aureus | Absent in 10 ml | BDS 1869 | 500.00 | 14 |  | 5 |  | 0.00 |
| 57. | 57(1). Skin Powder for babies (Type-1,talc based) (BDS 1844:2024) | Description | Skin powder for babies shall consist principally of a finely powdered, free flowing absorbent innocuous material such as natural talc and may contain a mild perfume and other raw materials. The material shall be free from boric acid. | BDS 1844:2024 | 100.00 | 14 | 10100.00 | 5 | 20200.00 | 0.00 |
|  |  | Matter insoluble in boiling water,%m/m | 90.00 (Min) | BDS 1844:2024, App-B | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Fineness: |  |  |  | 14 |  | 5 |  | 0.00 |
|  |  | a) Residue on 75 micronsieve,%m/m | 2.00 (Max) | BDS 1844:2024, App-C | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Residue on 150 micron sieve, %m/m | 0.50 (Max) | BDS 1844:2024, App-C | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Moisture and volatile matter, % m/m | 2.00 (Max) | BDS 1844:2024, App-D | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH of aqueous suspension | 5.5 to 8.00 | BDS 1844:2024, App-E | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metal (as Pb), ppm | 20.00 (Max) | BDS 1844:2024, App-F | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as AS2O3), ppm | 2.00 (Max) | BDS 1844:2024, App-G | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mercury (Hg), parts per million, Max. | 1.0 | BDS ISO/TR 17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Dermatological safety (use test) | It Shall not bring harmful effect to the body. | BDS 1844:2024 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Loss on Ignition, per cent by mass | 9.0 (Max) | BDS 1844:2024, App-H | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count, cfu/g, | 100 | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Yeast and mould count, CFU/g, Max | 100 | BDS ISO 16212 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Escherichia coli, per gram | Absent | BDS ISO 21150 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pseudomonas aeruginosa, per gram | Absent | BDS ISO 22717 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Staphylococcus aureus, per gram | Absent | BDS ISO 22718 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Candida albicans, per gram | Absent | BDS ISO 18416 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  | 57 (2). Skin Powder for babies (Type-2,starch based) (BDS 1844:2024) | Description | Skin powder for babies shall consist principally of a finely powdered, free flowing absorbent innocuous material such as natural talc and may contain a mild perfume and other raw materials. The material shall be free from boric acid. | BDS 1844:2024 | 100.00 | 14 | 10100.00 | 5 | 20200.00 | 0.00 |
|  |  | Matter insoluble in boiling water,%m/m | ---- | BDS 1844:2024, App-B | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Fineness: |  |  |  | 14 |  | 5 |  | 0.00 |
|  |  | a) Residue on 75 micronsieve,%m/m | 2.00 (Max) | BDS 1844:2024, App-C | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Residue on 150 micron sieve, %m/m | 0.50 (Max) | BDS 1844:2024, App-C | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Moisture and volatile matter, % m/m | 15.00 (Max) | BDS 1844:2024, App-D | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH of aqueous suspension | 5.5 to 8.00 | BDS 1844:2024, App-E | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metal (as Pb), ppm | 20.00 (Max) | BDS 1844:2024, App-F | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as AS2O3), ppm | 2.00 (Max) | BDS 1844:2024, App-G | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mercury (Hg), parts per million, Max. | 1.0 | BDS ISO/TR 17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Dermatological safety (use test) | It Shall not bring harmful effect to the body. | BDS 1844:2024 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Loss on Ignition, per cent by mass | 98.0 (Min) | BDS 1844:2024, App-H | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count, cfu/g, | 100 | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Yeast and mould count, CFU/g, Max | 100 | BDS ISO 16212 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Escherichia coli, per gram | Absent | BDS ISO 21150 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pseudomonas aeruginosa, per gram | Absent | BDS ISO 22717 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Staphylococcus aureus, per gram | Absent | BDS ISO 22718 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Candida albicans, per gram | Absent | BDS ISO 18416 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  | 57(3). Skin Powder for babies (Type-3, combined Starch+talc) (BDS 1844:2024) | Description | Skin powder for babies shall consist principally of a finely powdered, free flowing absorbent innocuous material such as natural talc and may contain a mild perfume and other raw materials. The material shall be free from boric acid. | BDS 1844:2024 | 100.00 | 14 | 10100.00 | 5 | 20200.00 | 0.00 |
|  |  | Matter insoluble in boiling water,%m/m | (Talc content × 0.9) | BDS 1844:2024, App-B | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Fineness: |  |  |  | 14 |  | 5 |  | 0.00 |
|  |  | a) Residue on 75 micronsieve,%m/m | 2.00 (Max) | BDS 1844:2024, App-C | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Residue on 150 micron sieve, %m/m | 0.50 (Max) | BDS 1844:2024, App-C | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Moisture and volatile matter, % m/m | a)Talc content × 0.02) + [(100 -Talc content) × 0.15] or b) (Starch content × 0.15) + [(100 - Starch content) × 0.02] | BDS 1844:2024, App-D | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH of aqueous suspension | 5.5 to 8.00 | BDS 1844:2024, App-E | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metal (as Pb), ppm | 20.00 (Max) | BDS 1844:2024, App-F | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as AS2O3), ppm | 2.00 (Max) | BDS 1844:2024, App-G | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mercury (Hg), parts per million, Max. | 1.00 | BDS ISO/TR 17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Dermatological safety (use test) | It Shall not bring harmful effect to the body. | BDS 1844:2024 | 100.00 | 14 |  | 5 |  | 0.00 |
|  |  | Loss on Ignition, per cent by mass | --- | BDS 1844:2024, App-H | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count, cfu/g, | 100 | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Yeast and mould count, CFU/g, Max | 100 | BDS ISO 16212 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Escherichia coli, per gram | Absent | BDS ISO 21150 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pseudomonas aeruginosa, per gram | Absent | BDS ISO 22717 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Staphylococcus aureus, per gram | Absent | BDS ISO 22718 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Candida albicans, per gram | Absent | BDS ISO 18416 | 1000.00 | 14 |  | 5 |  | 0.00 |
| 58. | Skin Cream and Lotion for baby (BDS 1858:2019) | Description | The skin lotions shall be a homogeneous emulsion or unctuous mass, with a pleasant odour. It shall be white or pigmented or of uniform colour. | BDS 1858:2019. | 100.00 | 14 | 5000.00 | 5 | 10000.00 | 0.00 |
|  |  | Thermal stability at 45oC (Min) | To pass the test | BDS 1420, App-D-1 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH | 5.00 to 8.00 | BDS 1420, App-D-2 | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Non Volatile Matter at 105oC, % m/m | 10.00 (Min) | BDS 1420, App-B | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Water content, %m/m | 90 (Max) | BDS 1420,App-D-4 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Peroxide Value, mg/Kg | 10 (Max) | - | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Aerobic Plate, Count/gm | 500 (Max) | BDS 1869 | 800.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pseudomonas aeruginosa | Absent in 10 gm | BDS 1869 | 800.00 | 14 |  | 5 |  | 0.00 |
|  |  | Staphylococcus aureus | Absent in 10 gm | BDS 1869 | 800.00 | 14 |  | 5 |  | 0.00 |
| 59. | Baby Shampoo (BDS 1884:2024) | Description | The sample shall be in the form of a liquid, emulsion or paste. It may be colored and perfumed. | BDS 1884:2024 | 100.00 | 14 | 12600.00 | 5 | 25200.00 | 0.00 |
|  |  | Active synthetic anionic ingredient content, % m/m | 3.00 (Min) | BDS 1884:2024, App-B | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH at 27 ± 2 O C | 6.0 – 7.5 | BDS 1884:2024, App-C | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Inorganic Salts, % by mass | 3.00 (Max) | BDS 1884:2024 , App-C | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lather Volume for one percent solution | 100.00 mm (Min) | BDS 1884:2024 , App-E | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Formaldehyde content | Absent | BDS 1884:2024 , App-F | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Non volatile alcohol soluble matter, per cent by mass, min. | 4.0 (Min) | BDS 1884:2024 , App-G | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals (as Pb), parts per million, Max | 20 (Max) | BDS 1884:2024 , App-G | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), parts per million, Max | 1.5 (Max) | BDS ISO/TR 17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total Combined Yeast and Mould count, CFU/mL, Max. | 100.00 (Max) | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Escherichia coli, per gram or mL | 100.00 (Max) | BDS ISO 16212 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Candida albicans, per gram or mL | Absent | BDS ISO 21150 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Staphylococcus aureus, per gram | Absent | BDS ISO 22717 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pseudomonas aeruginosa, per gram or m | Absent | BDS ISO 22718 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total Aerobic Microbial Count, CFU/mL, Max | Absent | BDS ISO 184416 | 1000.00 | 14 |  | 5 |  | 0.00 |
| 60. | 60(1) Enamel Synthetic Exterior (Undercoating) (BDS 1423:2018) | Viscosity by ford cup No.4 at 30 ± 20C, | 80.0 ( Min) | ASTM D 1200 | 1500.00 | 14 | 6800.00 | 5 | 13600.00 | 0.00 |
|  |  | Drying time , hours |  | IS 101Part-3/sec 1 | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | a) Surface dry | 3 hours (Max) | IS 101Part-3/sec 1 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Hard dry | 24 hours (Max) | IS 101 Part-3/sec 1 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | c) Tack Free | 8 hours (Max) | IS101 Part-3/sec 1 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Fineness of grind in Heghmangauge(HG) | Not less than 4 | IS 101 Part-3/sec 5 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Flash point | Not below 300C | IS 101 Part-1/sec 6 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Accelerated Storage stability test , at 600C, 96h | Shall pass the test | BDS 1423:2018 App-F | 800.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead content (as Pb), ppm | 90.00 (Max) | BDS ISO 6503 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  | 60(2) Enamel Synthetic Exterior (Undercoating/Finising) (BDS 1423:2018 | Viscosity by ford cup No.4 at 30 ± 20C, | 80.0 ( Min) | ASTM D 1200 | 1500.00 | 14 | 6800.00 | 5 | 13600.00 | 0.00 |
|  |  | Drying time , hours |  | IS 101Part-3/sec 1 | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | a) Surface dry | 6 hours (Max) | IS 101Part-3/sec 1 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Hard dry | 24 hours (Max) | IS 101 Part-3/sec 1 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | c) Tack Free | 12 hours (Max) | IS101 Part-3/sec 1 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Fineness of grind in Heghmangauge(HG) | Not less than 6 | IS 101 Part-3/sec 5 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Flash point | Not below 300C | IS 101 Part-1/sec 6 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Accelerated Storage stability test , at 600C, 96h | Shall pass the test | BDS 1423:2018 App-F | 800.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead content (as Pb), ppm | 90.00 (Max) | BDS ISO 6503 | 1000.00 | 14 |  | 5 |  | 0.00 |
| 61. | 61(1) Emulsion Paint (Type – 1 (Class A and B) (BDS 1827:2018) | Drying time, hours |  |  | 0.00 | 14 | 5600.00 | 5 | 11200.00 | 0.00 |
|  |  | a) Surface dry Class C | 45.0 minutes (Max) | BDS 1827:2018. B1.1 | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Hard dry Class C | 4.0 hours (Max) | BDS 1827:2018. B1.2 | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Finish | Smooth and uniform | BDS 1827:2018. B1.3 | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Colour/Shade | As per customer choice |  | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Viscosity at 30 ± 20 | 85 ku (Min) | ASTM D1200 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Resistance to wet abrasion | To pass the test | BDS 1827:2018. Ann-C | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Temperature stability | To pass the test | BDS 1827:2018. Ann-D | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Opacity | To pass the test | BDS 1827:2018. Ann-A | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead content (as dry film), ppm | 90.00 (Max) | BDS ISO 6503 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  | 61(2) Emulsion Paint (Type – 1 (Class C and D) (BDS 1827:2018) | Drying time, hours |  |  | 0.00 | 14 | 5600.00 | 5 | 11200.00 | 0.00 |
|  |  | a) Surface dry Class C | 90.0 minutes (Max) | BDS 1827:2018. B1.1 | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Hard dry Class C | 8.0 hours (Max) | BDS 1827:2018. B1.2 | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Finish | Smooth and uniform | BDS 1827:2018. B1.3 | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Colour/Shade | As per customer choice |  | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Viscosity at 30 ± 20 | 85 ku (Min) | ASTM D1200 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Resistance to wet abrasion | To pass the test | BDS 1827:2018. Ann-C | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Temperature stability | To pass the test | BDS 1827:2018. Ann-D | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Opacity | To pass the test | BDS 1827:2018. Ann-A | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead content (as dry film), ppm | 90.00 (Max) | BDS ISO 6503 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  | 61(3) Emulsion Paint (Type – 2 (Class A and B) (BDS 1827:2018) | Drying time, hours |  |  | 0.00 | 14 | 5600.00 | 5 | 11200.00 | 0.00 |
|  |  | a) Surface dry Class C | 45.0 minutes (Max) | BDS 1827:2018. B1.1 | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Hard dry Class C | 4.0 hours (Max) | BDS 1827:2018. B1.2 | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Finish | Smooth and uniform | BDS 1827:2018. B1.3 | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Colour/Shade | As per customer choice |  | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Viscosity at 30 ± 20 | 85 ku (Min) | ASTM D1200 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Resistance to wet abrasion | To pass the test | BDS 1827:2018. Ann-C | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Temperature stability | To pass the test | BDS 1827:2018. Ann-D | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Opacity | To pass the test | BDS 1827:2018. Ann-A | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead content (as dry film), ppm | 90.00 (Max) | BDS ISO 6503 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  | 61(4) Emulsion Paint (Type – 2 (Class C and D) (BDS 1827:2018) | Drying time, hours |  |  | 0.00 | 14 | 5600.00 | 5 | 11200.00 | 0.00 |
|  |  | a) Surface dry Class C | 90.0 minutes (Max) | BDS 1827:2018. B1.1 | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Hard dry Class C | 8.0 hours (Max) | BDS 1827:2018. B1.2 | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Finish | Smooth and uniform | BDS 1827:2018. B1.3 | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Colour/Shade | As per customer choice |  | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Viscosity at 30 ± 20 | 85 ku (Min) | ASTM D1200 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Resistance to wet abrasion | To pass the test | BDS 1827:2018. Ann-C | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Temperature stability | To pass the test | BDS 1827:2018. Ann-D | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Opacity | To pass the test | BDS 1827:2018. Ann-A | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead content (as dry film), ppm | 90.00 (Max) | BDS ISO 6503 | 1000.00 | 14 |  | 5 |  | 0.00 |
| 62. | Economy Emulsion Paint (Distemper) (BDS 1833:2018) | Drying time , hours |  |  | 0.00 | 14 | 5600.00 | 5 | 11200.00 | 0.00 |
|  |  | a) Surface dry | 1 hours (Max) | BDS 1833:2018 A1.1 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Hard dry | 6 hours (Max) | BDS 1833:2018 A1.2 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Finish | Smooth & Uniform | BDS 1833:2018 A1.3 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Color/Shade | Visual | BDS 1833:2018Apx-B | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Temperature Stability | To pass the test | BDS 1833:2018Apx-C | 2000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead content (as Pb), ppm | 90.00 (Max) | BDS ISO 6503 | 1000.00 | 14 |  | 5 |  | 0.00 |
| 63. | 63(1). Natural Henna/Mehedi (Powder) (BDS 1966: 2019) | Description | The product shall be in the form of fine dried powder obtained from fresh leaves of henna plant. It shall be free from extraneous adulterants. | BDS 1966: 2019 | 100.00 | 14 | 11000.00 | 5 | 22000.00 | 0.00 |
|  |  | Moisture and volatile matter,% m/m | 10.00 (Max) | 4 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cold water extract,% m/m | 25.0-32.0 | 5 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Crude fibre, % m/m | 10.0-15.0 | 6 | 700.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mineral matter, % m/m | 5.0-12.0 | 7 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Acid insoluble ash,% m/m | 3.0-6.0 | 8 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Extraneous sand,% m/m | 5.0 (Max) | 9 | 700.00 | 14 |  | 5 |  | 0.00 |
|  |  | Presence of extraneous dyes | To pass the test | 10 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lawsone Pigment, % m/m | 1.0 (Min) | 11 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH value at 27±2 ˚C (for powder 20.0 % solution & directly for paste) | 5.0-7.0 | 12 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals (as Pb), ppm | 20.00 (Max) | BDS ISO/TR 17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), ppm | 2.00 (Max) | BDS 1333 BDS ISO/TR 17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count, cfu/g | 1000 | BDS 1869 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Gram negative pathogens | Less than 10 | BDS 1869 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  | 63(2). Natural Henna/ Mehedi (Paste) (BDS 1966: 2019) | Description | The product shall be in the form of fine dried powder obtained from fresh leaves of henna plant. It shall be free from extraneous adulterants. | BDS 1966: 2019 | 100.00 | 14 | 11000.00 | 5 | 22000.00 | 0.00 |
|  |  | Moisture and volatile matter,% m/m | 90.00 (Max) | 4 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cold water extract,% m/m | 5.0 (Min) | 5 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Crude fibre, % m/m | 1.0 (Min) | 6 | 700.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mineral matter, % m/m | 2.0 (Max) | 7 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Acid insoluble ash,% m/m | 1.0 (Max) | 8 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Extraneous sand,% m/m | 1.0 | 9 | 700.00 | 14 |  | 5 |  | 0.00 |
|  |  | Presence of extraneous dyes | To pass the test | 10 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lawsone Pigment, % m/m | 0.5 (Min) | 11 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH value at 27±2 ˚C (for powder 20.0 % solution & directly for paste) | 5.0-7.0 | 12 | 200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals (as Pb), ppm | 20.00 (Max) | BDS ISO/TR 17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), ppm | 2.00 (Max) | BDS 1333 BDS ISO/TR 17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | a) Total viable count, cfu/g | 1000 | BDS 1869 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | b) Gram negative pathogens | Less than 10 | BDS 1869 | 1500.00 | 14 |  | 5 |  | 0.00 |
| 64. | Household Dish Washing Liquid (BDS 1554: 2021) | Description | The sample shall be homogeneous liquid, free from foreign matter. | BDS 1554: 2021 | 100.00 | 14 | 4500.00 | 4 | 9000.00 | 0.00 |
|  |  | Odour | Neither the detergent nor a solution of the detergent in water up to 60 °C should have an objectionable odour. | BDS 1554: 2021 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Residual Test | The detergent shall not leave any residual taste on washed articles. | BDS 1554: 2021 | 400.00 | 14 |  | 4 |  | 0.00 |
|  |  | Rinsing properties | Detergent shall be free rinsing. | Annex-C | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | pH, 1.0% (v/v) solution in water at (27 ± 2) ºC | 4.50-9.50 | Annex-B | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Surfactants as SLES, % m/m | 12.00 (not less than) | Annex D and ISO 2271 | 1500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Viscosity, CPS at 20 ºC (Brooke field, RPM-30, Spindle-62)), | 325 (Min) | Annex E | 1000.00 | 14 |  | 4 |  | 0.00 |
| 65. | Liquid Floor Cleaner (BDS 1859: 2021) | Description | The product shall be a stable uniform solution of surfactant. It shall not contain any abrasive and shall be non-irritating to the skin | BDS 1859: 2021 | 100.00 | 14 | 6600.00 | 4 | 13200.00 | 0.00 |
|  |  | pH, 1.0% (v/v) solution in distilled water at (27 ± 2) ºC | 4.50-10.50 | BDS 1859: 2021, App-A | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Solubility in distilled water | The product shall be completely soluble in distilled water | BDS 1859: 2021, App-B | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Solubility in in water having a hardness of 50 ppm | The product shall be completely soluble in water having a hardness of 50 ppm | BDS 1859: 2021, App-B | 600.00 | 14 |  | 4 |  | 0.00 |
|  |  | Rinsing Properties | The product shall be free-rinsing | BDS 1859: 2021, App-C | 700.00 | 14 |  | 4 |  | 0.00 |
|  |  | Cleaning efficiency of 1.0% (v/v) solution, Percent by volume | 80.0 (Min) | BDS 1859: 2021, App-G | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Deleterious action on painted surfaces not greater than | One-half the loss of 60 degree specular gloss | BDS 1859: 2021, App-E | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Deleterious action on linoleum, rubber and vinyl tile flooring | No softening, swelling, cracking, bleaching and discoloration | BDS 1859: 2021, App-F | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Surfactant Content, %m/m | 3.00 (min) | BDS 1859: 2021, App-G | 1200.00 | 14 |  | 4 |  | 0.00 |
| 66. | 66 (1) Liquid Toilet Cleaner (Type-Acidic) (BDS 1707: 2021) | Description | The material shall be homogeneous viscous liquid and it any may be suitably colored. The material shall contain hydrochloric acid and any necessary wetting agents, inhibitors and other desirable additives. | BDS 1707: 2021 | 100.00 | 14 | 5300.00 | 4 | 10600.00 | 0.00 |
|  |  | Total Acidity (as HCl or H3PO4), percent by mass | 7-12 | BDS 1707: 2021, App-A | 1200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Sediment content, % m/v | 0.50 (Max) | BDS 1707: 2021, App-B | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Efficacy test |  |  | 0.00 | 14 |  | 4 |  | 0.00 |
|  |  | a)Ability to remove rust & stains within 5 minutes | To pass the test | BDS 1707: 2021, App-C | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | b)Ability to remove lime scale: Mass of loss of marble, % m/m | 0.5 (Min) | BDS 1707: 2021, App-D | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Effect on porcelain Enamel | shall not any etching effect | BDS 1707: 2021, App-E | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Corrotional Inhibition | 0.15 (Max) | BDS 1707: 2021, App-F | 1000.00 | 14 |  | 4 |  | 0.00 |
|  | 66 (2) Lquid Toilet Cleaner (Type-Alkaline) (BDS 1707: 2021) | Description | The material shall be homogeneous viscous liquid and it any may be suitably colored. The material shall contain hydrochloric acid and any necessary wetting agents, inhibitors and other desirable additives. | BDS 1707: 2021 | 100.00 | 14 | 5300.00 | 4 | 10600.00 | 0.00 |
|  |  | PH | 10.5 - 13 | BDS 1707: 2021, App-G | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Surfactant as SLES, % m/m | 5.00 (Min) | BDS 1707: 2021, App-H | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Viscosity, cps | 350 (Min) | BDS 1707: 2021, App-I | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Sediment content, % m/v | 0.10 (Max) | BDS 1707: 2021, App-B | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Efficacy test |  |  | 0.00 | 14 |  | 4 |  | 0.00 |
|  |  | a)Ability to remove rust & stains within 5 minutes | To pass the test | BDS 1707: 2021, App-C | 750.00 | 14 |  | 4 |  | 0.00 |
|  |  | b)Ability to remove lime scale: Mass of loss of marble, % m/m | 0.50 (Min) | BDS 1707: 2021, App-D | 750.00 | 14 |  | 4 |  | 0.00 |
|  |  | Effect on porcelain Enamel | shall not any etching effect | BDS 1707: 2021, App-E | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Corrotional Inhibition | 0.15 (Max) | BDS 1707: 2021, App-F | 1000.00 | 14 |  | 4 |  | 0.00 |
|  | 66 (3) Liquid Toilet Cleaner (Type-Neutral) (BDS 1707: 2021) | Description | The material shall be homogeneous viscous liquid and it any may be suitably colored. The material shall contain hydrochloric acid and any necessary wetting agents, inhibitors and other desirable additives. | BDS 1707: 2021 | 100.00 | 14 | 5300.00 | 4 | 10600.00 | 0.00 |
|  |  | PH | 6.5 – 7.5 | BDS 1707: 2021, App-G | 200.00 | 14 |  | 4 |  | 0.00 |
|  |  | Surfactant as SLES, % m/m | 5.00 (Min) | BDS 1707: 2021, App-H | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Viscosity, cps | 200 (Min) | BDS 1707: 2021, App-I | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Sediment content, % m/v | 0.10 (Max) | BDS 1707: 2021, App-B | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | Efficacy test |  |  | 0.00 | 14 |  | 4 |  | 0.00 |
|  |  | a)Ability to remove rust & stains within 5 minutes | To pass the test | BDS 1707: 2021, App-C | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | b)Ability to remove lime scale: Mass of loss of marble, % m/m | 0.50 (Min) | BDS 1707: 2021, App-D | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Corrotional Inhibition | 0.15 (Max) | BDS 1707: 2021, App-F | 1000.00 | 14 |  | 4 |  | 0.00 |
| 67. | Nail Polish (BDS 1421:1992) | Description | Nail polish shall be glossy on the nails after complete drying. Evaluation shall be made visually. Moreover, it shall not leave any stain on the nails after being removed with the aid of nail polish remover. | BDS 1421:1992 | 100.00 | 14 | 4500.00 | 3 | 9000.00 | 0.00 |
|  |  | Non-volatile matter, m/m% | 25.0 (Min) | A-2 | 1200.00 | 14 |  | 3 |  | 0.00 |
|  |  | Drying time in minutes | 3.0 (Max) | A-3 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Adhesion test | The material shall be taken to have passed the test if not more than 10 percent of the film is peeled off. | A-4 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Scratch test | The material shall be considered to have passed the test if the pilot lamp does not glow throughout scrathing of the film. | A-5 | 600.00 | 14 |  | 3 |  | 0.00 |
|  |  | Blush test | The material shall be Taken to have passed the test if no ‘blush’ or a very slight ‘blush’ is observed at the edges of the film. | A-6 | 600.00 | 14 |  | 3 |  | 0.00 |
|  |  | Heavy metals | a) The material shall be considered to have passed the test if no black precipitate appears after passing hydrogen sulphide gas. b) The material shall be considered to have passed the test if no yellow precipitate appears after adding the potassium chromate solution. | A-7 | 1000.00 | 14 |  | 3 |  | 0.00 |
| 68. | Gold (BDS-1515 : 1995) | Gold, (as Au) % m/m | 99.50 (Min) (Fine Gold) | ISO11426:2014 ( Fire Assay) | 5000.00 | 5 | 5000.00 | 5 | 5000.00 | 0.00 |
| 69. | 69.(1) Liquid Hand Wash (Soap Based) (BDS 1973: 2019) | pH at 25±1 ˚C (For 10.0 % aqueous solution) | 10.0 (Max) | BDS ISO 4316 | 300.00 | 14 | 4500.00 | 5 | 9000.00 | 0.00 |
|  |  | Total fatty matter | 15.00 (Min) | 9 of BDS 406 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total free alkali, | 0.05 (Max) | BDS ISO 684 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Insoluble matter | 0.50 (Max) | 5 of BDS 406 | 1200.00 | 14 |  | 5 |  | 0.00 |
|  | 69.(2) Liquid Hand Wash (Surfactant based) (BDS 1973: 2019) | pH at 25±1 ˚C (For 10.0 % aqueous solution) | 4.0-10.0 | BDS ISO 4316 | 500.00 | 14 | 4500.00 | 5 | 9000.00 | 0.00 |
|  |  | Total surface active agents, %m/m | 7.0 (Min) | BDS 1973: 2019 Annex A | 4000.00 | 14 |  | 5 |  | 0.00 |
|  | 69.c)Liquid Hand Wash (Combination) (BDS 1973: 2019) | pH at 25±1 ˚C (For 10.0 % aqueous solution) | 4.0-10.0 | BDS ISO 4316 | 500.00 | 14 | 4500.00 | 5 | 9000.00 | 0.00 |
|  |  | Total surface active agents, %m/m | 7.0 (Min) | BDS 1973: 2019 Annex A | 4000.00 | 14 |  | 5 |  | 0.00 |
| 70. | Alcohol Based Hand Sanitizers (BDS 1980: 2020) | Appearance/ Description | The sanitizers shall have an acceptable odour, shall not have any disagreeable odour or smell. It shall be a clear, colour/colourless homogeneous and in the form of liquid or gel. It shall be free from visible suspended matter or precipitated contaminants. | Visual | 100.00 | 14 | 14100.00 | 5 | 18200.00 | 0.00 |
|  |  | Ethyl alcohol (Ethanol) / Isopropyl alcohol (IPA), %v/v | 70.0 (Min) | BDS 1980: 2020, Annex-A | 5000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Bactericidal/ Disinfectant efficacy | To pass the test | BDS 1980: 2020, Annex-B | 2000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Impurities |  | BDS 1980: 2020, Annex-A | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | Methanol | 250 (Max) | GC Method | 5000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pyridine | 80 (Max) |  | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | Benzene | 2 (Max) |  | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | Acetaldehyde | 50 (Max) |  | 0.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH | 5.0-8.0 | ASTM D 6423 | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Fragrance and Colour | As Required | BDS 1340 (P-1) and (P-2) | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Emollients, percent by volume | Optional | -- | 1200.00 | 14 |  | 5 |  | 0.00 |
| 71. | 71(1)Glass Tableware (Type: Flatware) (BDS 1874:2013) | Quality of glass table ware | The tableware shall be as free as possible from strain and visible defects. It shall be as far as possible free from any colour as seen transversely through the body, except in the case of coloured or decorated articles. | BDS 1874:2013, Cl – 3.1.1 | 300.00 | 14 | 6300.00 | 5 | 12600.00 | 0.00 |
|  |  | Limit of Alkalinity: Class HGB 4 | Consumption of 0.01 mol/L HCL, 0.85 up to and including 2.0 | IS 2303 (part 1/sec 1): 2012, ISO 719:1985 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  |  | equivalent of alkali, g of Na2O per g of glass grains, mean value (from 264 up to an including 620) | IS 2303 (part 1/sec 1): 2012, ISO 719:1985 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead, mg/dm2 | 0.80 (Max) | ISO 6486-1 and ISO 7086 - 1 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cadmium, mg/dm2 | 0.07 (Max) | ISO 6486-1 and ISO 7086 - 1 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  | 71(2)Glass Tableware (Type: Large Hollowware ) (BDS 1874:2013) | Quality of glass table ware | The tableware shall be as free as possible from strain and visible defects. It shall be as far as possible free from any colour as seen transversely through the body, except in the case of coloured or decorated articles. | BDS 1874:2013, Cl – 3.1.1 | 300.00 | 14 | 6300.00 | 5 | 12600.00 | 0.00 |
|  |  | Limit of Alkalinity: Class HGB 4 | Consumption of 0.01 mol/L HCL, 0.85 up to and including 2.0 | IS 2303 (part 1/sec 1): 2012, ISO 719:1985 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  |  | equivalent of alkali, g of Na2O per g of glass grains, mean value (from 264 up to an including 620) | IS 2303 (part 1/sec 1): 2012, ISO 719:1985 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead, mg/L | 0.75 (Max) | ISO 6486-1 and ISO 7086 - 1 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cadmium, mg/L | 0.25 (Max) | ISO 6486-1 and ISO 7086 - 1 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  | 71(3) Glass Tableware (Type: Small Hollowware) (BDS 1874:2013) | Quality of glass table ware | The tableware shall be as free as possible from strain and visible defects. It shall be as far as possible free from any colour as seen transversely through the body, except in the case of coloured or decorated articles. | BDS 1874:2013, Cl – 3.1.1 | 300.00 | 14 | 6300.00 | 5 | 12600.00 | 0.00 |
|  |  | Limit of Alkalinity: Class HGB 4 | Consumption of 0.01 mol/L HCL, 0.85 up to and including 2.0 | IS 2303 (part 1/sec 1): 2012, ISO 719:1985 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  |  | equivalent of alkali, g of Na2O per g of glass grains, mean value (from 264 up to an including 620) | IS 2303 (part 1/sec 1): 2012, ISO 719:1985 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead, mg/L | 1.5 (Max) | ISO 6486-1 and ISO 7086 - 1 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cadmium, mg/L | 0.5 (Max) | ISO 6486-1 and ISO 7086 - 1 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  | 71(4) Glass Tableware (Type: Storage Hollowware) (BDS 1874:2013) | Quality of glass table ware | The tableware shall be as free as possible from strain and visible defects. It shall be as far as possible free from any colour as seen transversely through the body, except in the case of coloured or decorated articles. | BDS 1874:2013, Cl – 3.1.1 | 300.00 | 14 | 6300.00 | 5 | 12600.00 | 0.00 |
|  |  | Limit of Alkalinity: Class HGB 4 | Consumption of 0.01 mol/L HCL, 0.85 up to and including 2.0 | IS 2303 (part 1/sec 1): 2012, ISO 719:1985 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  |  | equivalent of alkali, g of Na2O per g of glass grains, mean value (from 264 up to an including 620) | IS 2303 (part 1/sec 1): 2012, ISO 719:1985 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead, mg/L | 0.50 (Max) | ISO 6486-1 and ISO 7086 - 1 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cadmium, mg/L | 0.25 (Max) | ISO 6486-1 and ISO 7086 - 1 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  | 71(5) Glass Tableware (Type: Cups and Mugs ) (BDS 1874:2013) | Quality of glass table ware | The tableware shall be as free as possible from strain and visible defects. It shall be as far as possible free from any colour as seen transversely through the body, except in the case of coloured or decorated articles. | BDS 1874:2013, Cl – 3.1.1 | 300.00 | 14 | 6300.00 | 5 | 12600.00 | 0.00 |
|  |  | Limit of Alkalinity: Class HGB 4 | Consumption of 0.01 mol/L HCL, 0.85 up to and including 2.0 | IS 2303 (part 1/sec 1): 2012, ISO 719:1985 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  |  | equivalent of alkali, g of Na2O per g of glass grains, mean value (from 264 up to an including 620) | IS 2303 (part 1/sec 1): 2012, ISO 719:1985 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead, mg/L | 0.50 (Max) | ISO 6486-1 and ISO 7086 - 1 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cadmium, mg/L | 0.25 (Max) | ISO 6486-1 and ISO 7086 - 1 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  | 71(6) Glass Tableware (Type: Cooking ware) (BDS 1874:2013) | Quality of glass table ware | The tableware shall be as free as possible from strain and visible defects. It shall be as far as possible free from any colour as seen transversely through the body, except in the case of coloured or decorated articles. | BDS 1874:2013, Cl – 3.1.1 | 300.00 | 14 | 6300.00 | 5 | 12600.00 | 0.00 |
|  |  | Limit of Alkalinity: Class HGB 4 | Consumption of 0.01 mol/L HCL, 0.85 up to and including 2.0 | IS 2303 (part 1/sec 1): 2012, ISO 719:1985 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  |  | equivalent of alkali, g of Na2O per g of glass grains, mean value (from 264 up to an including 620) | IS 2303 (part 1/sec 1): 2012, ISO 719:1985 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead, mg/L | 0.50 (Max) | ISO 6486-1 and ISO 7086 - 1 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cadmium, mg/L | 0.05 (Max) | ISO 6486-1 and ISO 7086 - 1 | 1500.00 | 14 |  | 5 |  | 0.00 |
| 72. | Direct Moulded Sole (DMS) BDS 1555:2023 (1st Rev.) | Vamp, Quarter, Back Strap, Toe Cap and Socks. |  |  |  | 14 | 5100.00 | 5 | 10200.00 | 0.00 |
|  |  | 1. Chrome, %m/m | 2.60(Min) |  | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | 2. Oil & Fats, , %m/m | 1.3 to 1.5% |  | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | 3. Total Ash, %m/m | 2.5(Max) |  | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | 4. Water Proofing, % m/m (Water absorption after 2 hours) | 25 to 30 % (Max) |  | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Insole & Tongue |  |  |  | 14 |  | 5 |  | 0.00 |
|  |  | 5. Chrome, %m/m | 2.60(Min) |  | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | 6. Oil & Fats, , %m/m | 4 to 10% |  | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | 7. Total Ash, %m/m | 2.5(Max) |  | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | 8. Water Proofing, % m/m | 25 to 30 % (Max) |  | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | Toe puff and Stiffeners |  |  |  | 14 |  | 5 |  | 0.00 |
|  |  | 9. Oil & Fats, , %m/m | 4 9 (Max) |  | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | 10. Total Ash, %m/m | 1.5 (Max) |  | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | 11. Acid Insoluble Ash, %m/m | 0.7 |  | 400.00 | 14 |  | 5 |  | 0.00 |
|  |  | 12. pH | 3.5 (Min) |  | 200.00 | 14 |  | 5 |  | 0.00 |
| 73. | Rotational Modulated Polyethylene (Water Storage Tank ) BDS 1699:2002 | Test of water come in contact with the water tank material |  |  |  | 14 | 6000.00 | 14 | 6000.00 | 0.00 |
|  |  | Taste | Acceptable | BDS 1699:2022 | 500.00 | 14 |  | 14 |  | 0.00 |
|  |  | Colour | Normal water colour | BDS 1240:2001, APP-B | 500.00 | 14 |  | 14 |  | 0.00 |
|  |  | Odour | Acceptable | BDS 1414:2000, APP-B | 500.00 | 14 |  | 14 |  | 0.00 |
|  |  | Lead(Pb),ppm | Nil | BDS 1414:2000, APP-F | 1500.00 | 14 |  | 14 |  | 0.00 |
|  |  | Cadmium(Cd),ppm | Nil | BDS 1414:2000, APP-F | 1500.00 | 14 |  | 14 |  | 0.00 |
|  |  | Arsenic(As),ppm | Nil | Electro thermal AAS | 1500.00 | 14 |  | 14 |  | 0.00 |
| 74. | Food Grade Container (BDS 1958:2019) | 1.Environmental Stress-Crack Resistance | The container shall show no evidence of stress cracking or leakage after being kept in oven for 48 hours. |  | 500 | 28 | 28000.00 | 28 | 28000.00 | 0.00 |
|  |  | 2. Migration Test | The maximum extraction values for the container material shall not exceed 10 mg/dm2 or 60 mg/L at 400C for 10 days period. |  | 12000 | 28 |  | 28 |  | 0.00 |
|  |  | 3. Water Potability | Packaged mineral water and drinking water when stored in containers at 380C for 30 days, shall not acquire any unpleasant odour of bitter taste. |  | 500 | 28 |  | 28 |  | 0.00 |
|  |  | Migration of individual constituents: |  |  |  | 28 |  | 28 |  | 0.00 |
|  |  | 4. Acetaldehyde/Bisphenol A | 7 ppm/0.6ppm (Max) |  | 3500 | 28 |  | 28 |  | 0.00 |
|  |  | 5. Antimony (Sb) | 0.04 ppm, (Max) |  | 1000 | 28 |  | 28 |  | 0.00 |
|  |  | 6. Lead (Pb) | 2 ppm, (Max) |  | 1000 | 28 |  | 28 |  | 0.00 |
|  |  | 7. Cadmium (Cd) | 2 ppm, (Max) |  | 1000 | 28 |  | 28 |  | 0.00 |
|  |  | 8. Chromium (Cr) | 2 ppm, (Max) |  | 1000 | 28 |  | 28 |  | 0.00 |
|  |  | 9. Nickel (Ni) | 2 ppm, (Max) |  | 1000 | 28 |  | 28 |  | 0.00 |
|  |  | 10. Mercury (Hg) | 0.5 ppm, (Max) |  | 1000 | 28 |  | 28 |  | 0.00 |
| 75. | Water for use in Secondary Batteries (BDS 834:2007) | Description | The sample shall be free from suspended matter, odour, taste and shall be colourless when viewed through a depth of 300 mm. | BDS 834:2007 | 100.00 | 14 | 6600.00 | 3 | 13200.00 | 0.00 |
|  |  | Conductivity at 250C | 1.00 ms/m (Max) | AOAC Method 973.40 | 100.00 | 14 |  | 3 |  | 0.00 |
|  |  | Manganese (as Mn), mg/l | 0.1 (Max) | AOAC Method 974.27 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  |  | Residue on evaporation, mg/l | 150.00 (Max) | AOAC Method 973.40 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  |  | Ammoniacal Nitrogen expressed as N ,mg/l | 3.00 (Max) | BDS 834:2007,APP-B | 1000.00 | 14 |  | 3 |  | 0.00 |
|  |  | Nitrogen Oxide expressed as N ,mg/l | 20.00 (Max) | BDS 834:2007,APP-C | 800.00 | 14 |  | 3 |  | 0.00 |
|  |  | Chloride (as Cl), mg/l | 20.00 (Max) | AOAC Method 973.51 | 600.00 | 14 |  | 3 |  | 0.00 |
|  |  | Copper (as Cu), mg/l | 5.00 (Max) | AOAC Method 974.27 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  |  | Iron (as Fe), mg/l | 5.00 (Max) | AOAC Method 974.27 | 1000.00 | 14 |  | 3 |  | 0.00 |
| 76. | 76(1) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type-Viscosity Grade – SAE 0W) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 160 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt), 1000C, Min | 3.8 | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 100 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(2) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade – SAE 5W) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 160 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C, Min | 3.8 | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 100 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(3) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade – SAE 10W) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 190 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C, Min | 4.1 | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 100 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(4) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade – SAE 15W) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 190 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C, Min | 5.6 | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 95 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(5) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade – SAE 20W) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C, Min | 5.6 | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 95 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(5) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade – SAE 25W) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C, Min | 9.3 | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 95 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(6) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade – SAE 8) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 4.0 - < 6.1 | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 95 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(7) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade – SAE 12) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 5.0 – < 7.1 | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 95 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(8) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade – SAE 16) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 6.1 - < 8.2 | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 95 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(9) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade – SAE 20) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 6.9 - < 9.3 | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 95 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(10) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade – SAE 30) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 215 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 9.3 - < 12.5 | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 95 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(11) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade – SAE 40) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 215 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 12.5 - < 16.3 | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 90 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(12) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade – SAE 50) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 220 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 16.3 - < 21.9 | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 90 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(13) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade – SAE 60) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 220 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 21.9 - < 26.1 | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 90 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(14) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade SAE 0W - 20) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C, | 3.8 (Min for SAE 0W) | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 6.9 – < 9.3 (for SAE 20) |  |  | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 110 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(15) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade SAE 0W - 30) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C, | 3.8 (Min for SAE 0W) | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 9.3 – < 12.5 (for SAE 30) |  |  | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 110 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(16) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade SAE 0W - 40) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C, | 3.8 (Min for SAE 0W) | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 12.5 – < 16.3 (for SAE 40) |  |  | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 110 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(17) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade SAE 0W - 50) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C, | 3.8 (Min for SAE 0W) | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 16.3 – < 21.9 (for SAE 50) |  |  | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 110 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(18) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade SAE 5W - 20) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 3.8 (Min for SAE 5W) | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 6.9 – < 9.3 (for SAE 20) |  |  | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 110 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(19) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade SAE 5W - 30) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 3.8 (Min for SAE 5W) | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 9.3 – < 12.5 (for SAE 30) |  |  | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 110 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(20) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade SAE 5W - 40) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 3.8 (Min for SAE 5W) | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 12.5 – < 16.3 (for SAE 40) |  |  | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 110 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(21) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade SAE 5W - 50) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 3.8 (Min for SAE 5W) | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 16.3 – < 21.9 (for SAE 50) |  |  | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 110 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(22) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade SAE 5W - 60) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 3.8 (Min for SAE 5W) | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 21.9 – < 26.1 (for SAE 50) |  |  | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 110 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(23) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade SAE 10W - 30) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 4.1 (Min for SAE 10W) | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 9.3 – < 12.5 (for SAE 30) |  |  | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 110 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(24) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade SAE 10W - 40) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 4.1 (Min for SAE 10W) | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 12.5 – < 16.3 (for SAE 40) |  |  | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 110 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(25) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade SAE 10W - 50) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 4.1 (Min for SAE 10W) | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 16.3 – < 21.9 (for SAE 50) |  |  | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 110 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(25) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade SAE 10W - 60) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 4.1 (Min for SAE 10W) | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 21.9 – < 26.1 (for SAE 60) |  |  | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 110 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(26) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade SAE 20W - 30) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 5.6 (Min for SAE 20W) | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 9.3 – < 12.5 (for SAE 30) |  |  | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 110 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(27) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade SAE 20W - 40) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 5.6 (Min for SAE 20W) | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 12.5 – < 16.3 (for SAE 40) |  |  | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 110 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(28) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade SAE 20W - 50) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 5.6 (Min for SAE 20W) | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 16.3 – < 21.9 (for SAE 50) |  |  | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 110 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 76(29) Internal Combustion Engine Crankcase Oils for Automotive Application (Diesel and Gasoline) (Type- Viscosity Grade SAE 20W - 60) (BDS 343:2022) | Appearance | Oil shall be clear, bright and free from turbidity and sediment. | BDS 343:2012 | 300.00 | 14 | 6800.00 | 3 | 13600.00 | 0.00 |
|  |  | Density at 150C, g /ml | To be reported | ASTM D 4052 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Flash point COC,0C, Min | 200 | ASTM D 92 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Kinematic Viscosity, mm2/s / (cSt), 400C | To be reported | ASTM D 445 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Low -Shear-Rate Kinematic Viscosity, mm2/s / (cSt),1000C | 5.6 (Min for SAE 20W) | ASTM D 445/ASTM D 7042 | 1500.00 | 14 |  | 3 |  | 0.00 |
|  |  |  | 21.9 – < 26.1 (for SAE 60) |  |  | 14 |  | 3 |  | 0.00 |
|  |  | Viscosity-index, Min | 110 | ASTM D 2270 | 500.00 | 14 |  | 3 |  | 0.00 |
|  |  | Total acid No. mg KOH/g | To be reported | ASTM D 664 | 1000.00 | 14 |  | 3 |  | 0.00 |
|  | 77. Cement Paint (BDS 1706:2015) | Description | The material shall be in the form of finely divided powder, free from foreign matter and visible impurities. | BDS 1706:2015. | 100.00 | 14 | 1700.00 | 4 | 3400.00 | 0.00 |
|  |  | Drying time (Hard dry) | 24 hours (Max) | BDS 1706:2015; App-D | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Finish | The coating shall be smooth, hard, and firmly adherent and shall have a matt finish. | BDS 1706:2015; App-A | 100.00 | 14 |  | 4 |  | 0.00 |
|  |  | Residue on 63 micron BDS sieve (on dry material),percent by mass,Max | 5.0 | BDS 1706:2015; App-G | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Water repellency | Water absorbed by the panel is not more than 200 gm/m2 | BDS 1706:2015; App-H | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Pot life of mixed paint | Not less than one hour | BDS 1706:2015; App-I | 300.00 | 14 |  | 4 |  | 0.00 |
|  |  | Resistance to rubbing | The material shall be deemed to have passed the test if the test cambric cloth is not soiled by the film. | BDS 1706:2015; App-K | 300.00 | 14 |  | 4 |  | 0.00 |
| 78. | Face wash (BDS 2000:2022) | Description | The face wash shall be in the form of a liquid, emulsion, gel, paste and powder. Face wash may be coloured/colourless. It may also be perfumed or mild perfumed. | BDS 2000:2022, 4.1 | 100.00 | 14 | 13400.00 | 5 | 26800.00 | 0.00 |
|  |  | Surface Active Ingredients, Percent by Mass | 3.00 (Min) | BDS 2000:2022, A | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Stability at 45±1 °C for 48 hours phase separation | To pass the Test | BDS 2000:2022, B | 700.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH, 10% solution | 4-10 | BDS 2000:2022, C | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Plastic micro beads | Absent | BDS 2000:2022, D | 800.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals (as Pb), parts per million, | 20 (Max) | BDS ISO/TR 17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), parts per million, | 2 (Max) | BDS 1333 BDS ISO/TR 17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mercury (as Hg), parts per million, | 2 (Max) | BDS ISO/TR 17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count cfu/g | Not more than 1000 | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pathogenic bacteria, in 10 mL |  | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Candida albicans | Absent | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Staphyloccocus aureus | Absent | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Group A Streptococcus (GAS) | Absent | BDS 1869 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pseudomonas aeruginosa | Absent | BDS 1869 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Escherichia coli | Absent | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
| 79. | Household Insecticide Aerosol (BDS 1585:2023) | Knockdown, %, 20 min. | Flying insects = 50 (Min) Crawling insects = 50 (Min) Multipurpose = 50 (Min) | BDS 1585:2023, Annex A | 1000.00 | 14 | 9000.00 | 5 | 18000.00 | 0.00 |
|  |  | Mortality, %,24 h | 20 min. Flying insects = 80 (Min) Crawling insects = 80 (Min) Multipurpose = 80 (Min) | BDS 1585:2023, Annex A | 2000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Delivery rate at 30°C ± 5°C, g/s, | 5.0 (max.) | BDS 1585:2023, Annex B | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Leakage | To pass test | BDS 1585:2023, Annex C | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Particle size, μm | 10-100 | BDS 1585:2023, Annex D | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Internal pressure of the filled dispenser at 30°C ± 5°C, kPa, max. | 1000 | BDS 1585:2023, Annex E | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Clogging of dispenser valve | To pass test | BDS 1585:2023, Annex F | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | CFC | Absent |  | 1000.00 | 14 |  | 5 |  | 0.00 |
| 80. | Petroleum Jelly (BDS 1597:2023) | Description | Petroleum jelly shall be in the form of a translucent, soft mass, unctuous to the touch and retaining these characteristics on storage and also when melted and allowed to cool without stirring. It shall be not more than slightly fluorescent by daylight, even when melted. It may contain fragrances or herbal extracts/ powder. | BDS 1597:2023 | 100.00 | 14 | 10100.00 | 5 | 20200.00 | 0.00 |
|  |  | Solubility | It shall be insoluble in water; soluble in dichloromethane, chloromethane, diethyl ether, carbon disulfide and turpentine. It is slightly soluble in alcohol. | BDS 1597:2023 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Odour | The material shall be not of an objectionable odour at room temperature when rubbed on the skin. Further, the material when heated to 95 °C to 98 °C on a boiling water-bath for 30 minutes shall give no disagreeable odour. | BDS 1597:2023 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Melting range, ºC | 40 to 60 | BDS 1597:2023, A-2 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Specific Gravity at 60 ºC | 0.815 to 0.880 | BDS 1597:2023, A-3 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH | 5.0 to 8.0 | BDS 1597:2023, A-4 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Organic acids | To pass the test | BDS 1597:2023, A-5 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Sulphated ash, % m/m | 0.10(Max) | BDS 1597:2023, A-6 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Sulphur and sulphides | To pass the test | BDS 1597:2023, A-7 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Iodine value (Wijs), Max | 3.00 | BDS 1597:2023, A-8 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Light absorption (extinction of 0.1 percent solution), Max | 1.00 | BDS 1597:2023, A-9 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Volatile matter, % m/m | 5.0(Max) | BDS 1597:2023, A-10 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Lead (as Pb), ppm, Max. | 20.00 | BDS 1597:2023, A-11 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), ppm, Max. | 2.00 | BDS ISO/TR 17276 ISO 21392 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mercury (as Hg), ppm, Max. | 1.00 | BDS ISO/TR 17276 ISO 21392 | 1000.00 | 14 |  | 5 |  | 0.00 |
| 81. | Eye Care Products (BDS 1987:2023) | Glueing and War page of casing | The finished pencils shall be kept for 48 hours in a desiccator filled with water. The casing shall not separate. The warpage of casing in 80 percent of the sample shall not exceed 0.5 mm and in remaining 20 percent of the sample, shall not exceed 1.0 mm. | BDS 1987:2023, 8.2 | 500.00 | 14 | 8600.00 | 5 | 17200.00 | 0.00 |
|  |  | The wear of slip | (mg/100 cm2 area of paper) Min. 2 | BDS 1987:2023, 8.3 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Melting point, °C, (Except liquid eye care products) | 50 (Min) | BDS 1987:2023, B | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH, 10% dispersion | 6.5-7.5 | BDS 1987:2023, C | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Peroxide value, m.eq./1000 g, (Only for liquid eye care products) | 10 (Max) | BDS 1987:2023, D | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Freedom from grits | To pass the test | BDS 1987:2023, E | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic as As2O3 ppm, | 2.00 (Max) | BDS 1333 , F | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mercury (as Hg), parts per million, | 1.00 (Max) | BDS 1987:2023, G | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals (as Pb) ppm, | 20.00 (Max) | BDS 1987:2023, H | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count, CFU/g, Max | 1000 | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Gram negative Pathogens, CFU/g | Less than 10 | BDS 1869 | 1500.00 | 14 |  | 5 |  | 0.00 |
| 82. | Shaving Foam (BDS 1986:2021) | Description | Shaving foam shall be easily applied and free from any objectionable odour. It shall be white or pigmented and of uniform colour | BDS 1986:2021, 3.1 | 100.00 | 14 | 7800.00 | 5 | 15600.00 | 0.00 |
|  |  | Dermatological safety | Passes the tests | BDS 1986:2021, 3.4 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH of 10% solution at (27 ± 2) °C | 7.0-10.5 | BDS 1986:2021, A | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Internal Pressure, psig at 21 °C | 40-100 | BDS 1986:2021, B | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mercaptan, ppm, Max. | 5.00 (Max) | ASTM D 5305 ISO 3012 | 1200.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals as (Pb), ppm, Max. | 20.00 (Max) | BDS ISO/TR 17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), ppm, Max. | 2.00 (Max) | BDS 1333 BDS ISO/TR 17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count, cfu/g, | 1000 (Max.) | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Gram negative pathogen, cfu/g | Absent | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
| 83. | 83.(1) Oxidation Hair Dyes (Liquid/ Gel/ Cream) (Type-1) (BDS 1338 : 2023) | Dye |  |  |  | 14 | 4500.00 | 5 | 9000.00 | 0.00 |
|  |  | pH | 9.0 – 11.0 | 1338:2023- A | 250.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total Active Matter (Dye Content), Percent by Mass | 4.0 (Max) | 1338:2023 -B or D1 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals (as Pb), ppm | 20 (Max) | 1338:2023- F | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), ppm | 2 (Max) | 1338:2023- G | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Developer |  |  |  |  |  |  |  |  |
|  |  | pH | 1.8 to 4.0 | 1338:2023- A | 250.00 | 14 |  | 5 |  | 0.00 |
|  |  | Assay (as H2O2), percent (m/v) | 12 (Max) | 1338:2023- C | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals (as Pb), ppm | 20 (Max) | 1338:2023- F | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), ppm | 2 (Max) | 1338:2023 -G | 500.00 | 14 |  | 5 |  | 0.00 |
|  | 83.(2) Oxidation Hair Dyes (Liquid/ Gel/ Cream) (BDS 1338 : 2023) (Type-2) | Dye |  |  |  | 14 | 4500.00 | 5 | 9000.00 | 0.00 |
|  |  | pH | 9.0 – 11.0 | 1338:2023- A | 250.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total Active Matter (Dye Content), Percent by Mass | 3.0 (Max) | 1338:2023 -B or D1 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals (as Pb), ppm | 20 (Max) | 1338:2023- F | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), ppm | 2 (Max) | 1338:2023- G | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Developer |  |  |  |  |  |  |  |  |
|  |  | pH | 1.8 to 4.0 | 1338:2023- A | 250.00 | 14 |  | 5 |  | 0.00 |
|  |  | Assay (as H2O2), percent (m/v) | 12 (Max) | 1338:2023- C | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals (as Pb), ppm | 20 (Max) | 1338:2023- F | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), ppm | 2 (Max) | 1338:2023 -G | 500.00 | 14 |  | 5 |  | 0.00 |
|  | 83.(3) Oxidation Hair Dyes (Liquid/ Gel/ Cream) (BDS 1338 : 2023) (Type-3) | Dye |  |  |  | 14 | 4500.00 | 5 | 9000.00 | 0.00 |
|  |  | pH | 9.0 – 11.0 | 1338:2023- A | 250.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total Active Matter (Dye Content), Percent by Mass | 4.0 (Max) | 1338:2023 -B or D1 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Dye Ingredients | Present | 1338:2023 -E or D1 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals (as Pb), ppm | 20 (Max) | 1338:2023- F | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), ppm | 2 (Max) | 1338:2023- G |  |  |  |  |  |  |
|  |  | Developer |  |  |  |  |  |  |  |  |
|  |  | pH | 1.8 to 4.0 | 1338:2023- A | 250.00 | 14 |  | 5 |  | 0.00 |
|  |  | Assay (as H2O2), percent (m/v) | 12 (Max) | 1338:2023- C | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals (as Pb), ppm | 20 (Max) | 1338:2023- F | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), ppm | 2 (Max) | 1338:2023 -G | 500.00 | 14 |  | 5 |  | 0.00 |
| 84. | Shoe polish, Liquid (BDS 1589:1998) | Colour | The material shall be manufactured in conventional colour | BDS 1589:1998 | 500.00 | 14 | 4000.00 | 5 | 8000.00 | 0.00 |
|  |  | Consistency | The material shall be easily pourable, free flowing, homogeneous liquid, free from any grits and sedimentation. It shall have no tendency for the separation or crystallization of the constituent materials. | BDS 1589:1998, Appendix-A | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Applicability | The polish shall spread evenly on the surface without any mottling. The applied film shall not show any tendency while still being wet. | BDS 1589:1998, Appendix-B | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Non Volatile Matter, %m/m | 20.00 (Min) | BDS 1589:1998, Appendix-C | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Ash of non-volatile matter, %m/m | 3.00 (Max) | BDS 1589:1998, Appendix-D | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH | 6.5 to 9.0 | BDS 1589:1998, Appendix-E | 500.00 | 14 |  | 5 |  | 0.00 |
| 85. | Disposable Diaper (BDS 2006:2022) | Total viable bacterial count, per gram | 600 (Max) | BDS 2006:2022, C– 4.1 | 1000.00 | 14 | 1000.00 | 5 | 2000.00 | 0.00 |
| 86. | 86.(1) Face pack (Type-1) (BDS 1977:2020 | Description | The sample shall be a smooth paste or fluid or dry powder free from any gritty particles. It may be coloured for aesthetics and may contain natural or synthetic scrubbing materials. | BDS 1977:2020 | 100.00 | 14 | 7100.00 | 5 | 14200.00 | 0.00 |
|  |  | pH | 5-9 | A | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Stability at 45±1°C for 48 hours phase separation | Stable for two days (Min) | B | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Solid content (residue on evaporation),%,m/m | 10.0(Min) | C | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Loss on drying, Percent by mass | - | C | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Ash content, Percent by mass | - | D | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals(as Pb), ppm | 20.00 (Max) | BDS ISO/TR17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as AS2O3), ppm | 2.00 (Max) | BDS 1333 /BDS ISO/TR 17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count cfu/gm | Not more than 1000 | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Gram negative pathogens | Less than 10 | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  | 86.(2) Face pack (Type-2) (BDS 1977:2020 | Description | The sample shall be a smooth paste or fluid or dry powder free from any gritty particles. It may be coloured for aesthetics and may contain natural or synthetic scrubbing materials. | BDS 1977:2020 | 100.00 | 14 | 7100.00 | 5 | 14200.00 | 0.00 |
|  |  | pH | 5-9 | A | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Stability at 45±1°C for 48 hours phase separation | To pass the test | B | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Solid content (residue on evaporation), %,m/m | - | C | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Loss on drying, Percent by mass | 5.0(Max) | C | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Ash content, Percent by mass | 85.0(Min) | D | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals(as Pb), ppm | 20.00 (Max) | BDS ISO/TR17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as AS2O3), ppm | 2.00 (Max) | BDS 1333 /BDS ISO/TR 17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count cfu/gm | Not more than 1000 | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Gram negative pathogens | Less than 10 | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
| 87. | Kajol (BDS 1949:2018) | Description | The kajal shall be homogenous waxy mass, moulded either in pots, stick or pencil form. It shall be black in shade and shall be reasonably free from sweating. | BDS 1949:2018. | 100.00 | 14 | 6000.00 | 5 | 12000.00 | 0.00 |
|  |  | Melting point, oC, (Except liquid eye care products) | 50.00 (Min) | BDS 1949:2018, Annex-A | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Peroxide value, m.eq./1000 g | 10.00 (Max) | BDS 1949:2018, Annex-B | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Freedom from grits | To pass the test | BDS 1949:2018, Annex-C | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as AS2O3), ppm | 2.00 (Max) | BDS 1949:2018, Annex-D | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals(as Pb), ppm | 20.00 (Max) | BDS 1949:2018, Annex-E | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH, 10% dispersion | 6.50 to 7.50 | BDS 1949:2018, Annex-F | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count Cfu/gm | Not more than 1000 | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Gram negative pathogens | Less than 10 | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
| 88. | Transparent Toilet Soap (BDS 1908:2016) | Description | Transparent toilet soap shall be a saponified soap of firm and smooth texture. It shall be transparent, perfumed and shall possess good cleaning and lathering properties. | BDS 1908:2016 | 100.00 | 14 | 7200.00 | 5 | 14400.00 | 0.00 |
|  |  | Total fatty matter, %m/m | 35.00 (Min) | BDS 406, Annex-9 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Rosin acid, %m/m | 20.00 (Max) | BDS 406, Annex-14 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Unsaponified fatty matter , %m/m | 0.50 (Max) | BDS 406, Annex-13 | 900.00 | 14 |  | 5 |  | 0.00 |
|  |  | Free caustic alkali (as NaOH), ,% m/m | 0.05 (Max) | BDS 406, Annex-6 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Transparency | To pass the test | BDS 1908:2016, Annex-A | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Grittiness | To pass the test | BDS 1908:2016, Annex--B | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Synthetic detergent Content | Nil | .BDS 1445;App-B& C | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Phosphate Content | 5.0 (Max) | 20 of BDS 406 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Dermatological Safety | It shall not bring harmful effect to the body. | BDS 1824. | 200.00 | 14 |  | 5 |  | 0.00 |
| 89. | 89.(1) Glycerine Toilet Soap (Grade-1) (BDS 1536: 2016) | Description | Glycerine toilet soap shall be a saponified soap of firm and smooth texture. It can be transparent or non-transparent, perfumed and shall possess good cleaning and lathering properties. | BDS 1536:2016 | 100.00 | 14 | 7600.00 | 5 | 15200.00 | 0.00 |
|  |  | Synthetic detergent | Absent | BDS 1445 Annex-B and C | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Phosphate | Absent | BDS 406 Clause 20 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Dermatological safety | It shall not bring harmful effect to the body. | BDS 1824. | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total fatty matter, percent by mass, Min | 70.0 | BDS 406, Clause-9. | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Unsafonified fatty Matter, percent by mass, Max | Nil | BDS 406, Clause-13 | 800.00 | 14 |  | 5 |  | 0.00 |
|  |  | Free caustic alkali (as NaOH), percent by mass, Max | 0.05 | BDS 406, Clause-6.2 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Matter insoluble in alcohol, percent by mass, Max | 0.10 | BDS 406, Clause-5 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Chlorides (as NaCl) , percent by mass, Max | 0.75 | BDS 406, Clause-10 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total free alkali (as Na2O), percent by mass, Max | 0.10 | BDS 1536:2016, Annex-A | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Glycerine, percent by mass, Min | 5.00 | BDS 406, Clause-22 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Grittiness | To pass the test | BDS 1536:2016, Annex-B | 200.00 | 14 |  | 5 |  | 0.00 |
| 89. | 89.(2) Glycerine Toilet Soap (Grade-2) (BDS 1536: 2016) | Description | Glycerine toilet soap shall be a saponified soap of firm and smooth texture. It can be transparent or non-transparent, perfumed and shall possess good cleaning and lathering properties. | BDS 1536:2016 | 100.00 | 14 | 7600.00 | 5 | 15200.00 | 0.00 |
|  |  | Synthetic detergent | Absent | BDS 1445 Annex-B and C | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Phosphate | Absent | BDS 406 Clause 20 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Dermatological safety | It shall not bring harmful effect to the body. | BDS 1824. | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total fatty matter, percent by mass, Min | 54.0 | BDS 406, Clause-9. | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Unsafonified fatty Matter, percent by mass, Max | Nil | BDS 406, Clause-13 | 800.00 | 14 |  | 5 |  | 0.00 |
|  |  | Free caustic alkali (as NaOH), percent by mass, Max | 0.05 | BDS 406, Clause-6.2 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Matter insoluble in alcohol, percent by mass, Max | 0.40 | BDS 406, Clause-5 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Chlorides (as NaCl) , percent by mass, Max | 1.00 | BDS 406, Clause-10 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total free alkali (as Na2O), percent by mass, Max | 0.50 | BDS 1536:2016, Annex-A | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Glycerine, percent by mass, Min | 5.00 | BDS 406, Clause-22 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Grittiness | To pass the test | BDS 1536:2016, Annex-B | 200.00 | 14 |  | 5 |  | 0.00 |
| 90. | 90(1) Liquid Body Wash (Type-1: soap-based liquid body wash (BDS 1740:2024) | Total fatty matter, % (m/m), minimum | 5 | 9 of BDS 406 | 1000.00 | 14 | 6600.00 | 5 | 13200.00 | 0.00 |
|  |  | pH at 25±10C of 10% aqueous Solution, m/m | 4.0-9.0 | BDS ISO 4316: 2018 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total free alkali (as Na2O) | 0.05 | BDS ISO 684 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Matter insoluble in Ethanol, percent by mass, Max | 0.50 | 5 of BDS 406 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Aerobic Plate count per mL, Max | 1000 | ISO 21149 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pseudomonas aeruginosa | Absent | ISO 22717 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Staphylococcus aureus | Absent | ISO 22718 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  | 90(2) Liquid Body Wash (Type-2: synthetic surfactant-based liquid body wash) (BDS 1740:2024) | Total fatty matter, % (m/m), minimum | 5 | 9 of BDS 406 | 1500.00 | 14 | 6600.00 | 5 | 13200.00 | 0.00 |
|  |  | pH at 25±10C of 10% aqueous Solution, m/m | 4.0-9.0 | BDS ISO 4316: 2018 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Aerobic Plate count per mL, Max | 1000 | ISO 21149 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pseudomonas aeruginosa | Absent | ISO 22717 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Staphylococcus aureus | Absent | ISO 22718 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  | 90(3) Liquid Body Wash (Type-2: Synthetic Detergent and Soap (combination) based (BDS 1740:2024) | Total fatty matter, % (m/m), minimum | 5 | 9 of BDS 406 | 1500.00 | 14 | 6600.00 | 5 | 13200.00 | 0.00 |
|  |  | pH at 25±10C of 10% aqueous Solution, m/m | 4.0-9.0 | BDS ISO 4316: 2018 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Aerobic Plate count per mL, Max | 1000 | ISO 21149 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Pseudomonas aeruginosa | Absent | ISO 22717 | 1500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Staphylococcus aureus | Absent | ISO 22718 | 1500.00 | 14 |  | 5 |  | 0.00 |
| 91. | Hair Cream (BDS 1420:1993) | Description | The hair cream shall be in the form of a thick emulsion or unctuous mass. It shall be white or uniformly coloured, with or without perfume. | BDS 1420:1993 | 100.00 | 14 | 5400.00 | 5 | 10800.00 | 0.00 |
|  |  | Thermal stability | To pass the test | BDS 1420:1993, App D-1 | 800.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH | 5.0 to 9.0 | BDS 1420:1993, App D-2 | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total Fatty Subtance Content, %m/m | 15 (Min) | BDS 1420:1993, App D-3 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Water Content, %m/m | 85 (Max) | BDS 1420:1993, App D-4 | 700.00 | 14 |  | 5 |  | 0.00 |
|  |  | Test for rancidity | Shall be free from rancidity | BDS 1420:1993, App D-5 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total microbial count cfu/g, Max | 1000 (Max) | BDS 1420:1993, App-A | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mould count | 100 (Max) | BDS 1420:1993, App D-6 | 1000.00 | 14 |  | 5 |  | 0.00 |
| 92. | Non Oven wipes (BDS 2017: 2023) | Total viable count (cfu /gm), |  |  | 1000.00 | 14 | 1000.00 | 5 | 2000.00 | 0.00 |
| 93. | Synthetic Colour Paste (BDS 1967: 2019) | Description | The product shall be manufactured in paste form. It shall be in the form of a smooth, semisolid, homogeneous mass. It shall be free from extraneous adulterants. | BDS 1967: 2019 | 300.00 | 14 | 8700.00 | 5 | 17400.00 | 0.00 |
|  |  | Moisture and volatile matter,% m/m | 95.00 (Max) | BDS 1968, Cl-4 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Cold water extract,% m/m | 1.0 (Min) | BDS 1968, Cl-5 | 500.00 | 14 |  | 5 |  | 0.00 |
|  |  | Mineral matter, % m/m | 2.00 (Max) | BDS 1968, Cl-7 | 600.00 | 14 |  | 5 |  | 0.00 |
|  |  | Acid insoluble ash,% m/m | 1.0 (Max) | BDS 1968, Cl-8 | 750.00 | 14 |  | 5 |  | 0.00 |
|  |  | Extraneous sand,% m/m | 1.0 (Max) | BDS 1968, Cl-9 | 750.00 | 14 |  | 5 |  | 0.00 |
|  |  | Presence of extraneous dyes | To pass the test | BDS 1968, Cl-10 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | pH value at 27±2 ˚C (for 20.0 % solution) | 5.00-7.00 | BDS 1968, Cl-12 | 300.00 | 14 |  | 5 |  | 0.00 |
|  |  | Heavy metals (as Pb), ppm | 20.00 (Max) | BDS ISO/TR17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Arsenic (as As2O3), ppm | 2.00 (Max) | BDS 1333 BDS ISO/TR17276 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Total viable count, cfu/gm | 1000.0 (Max) | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
|  |  | Gram negative pathogens | Less than 10 | BDS 1869 | 1000.00 | 14 |  | 5 |  | 0.00 |
| 94. | Pencils (BDS 330: 2006) | #### Chemical Inertness of Pencil Markings | When a paper marked with pencil lines is dipped in hydrogen peroxide, it shall not be affected by the latter. | #### BDS 330:2006. CL-7.8 | 300.00 | 14 | 700.00 | 5 | 1400.00 | 0.00 |
|  |  | Toxicity test | The material used for making pencils e.g, lacquer, coating pigments should be non-toxic. | #### BDS 330:2006. CL-7.9 | 400.00 | 14 |  | 5 |  | 0.00 |
| 95. | Colour Pencils (BDS 2020:2023) (BDS 2020:2023) | a) For the Coating Material: Harmful Substance as coating material to be used for coating the casing, ppm (mg/kg), Max |  |  |  | 14 | 7500.00 | 4 | 15000.00 | 0.00 |
|  |  | i) Antimony (Sb) | 560.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | ii) Arsenic (As) | 47.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | iii) Barium (Ba) | 18750.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | iv) Cadmium (Cd) | 17.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | v) Lead (Pb) | 23.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | vi) Mercury (Hg) | 94.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | vii) Selenium (Se) | 460.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | viii) Chromium (Cr) | 60.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | b) For the Colour Pencil Lead: Hazardous Substances, ppm (mg/kg), Max |  |  |  | 14 |  | 4 |  | 0.00 |
|  |  | i) Antimony (Sb) | 45.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | ii) Arsenic (As) | 3.80 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | iii) Barium (Ba) | 1500.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | iv) Cadmium (Cd) | 1.30 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | v) Lead (Pb) | 2.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | vi) Mercury (Hg) | 7.50 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | vii) Selenium (Se) | 37.50 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | viii) Chromium (Cr) | 60.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  | 95.b) Colour Pencils (Type 2: Medium Rigid, Diameter of leads, 2.5mm (Min) (BDS 2020:2023) | a) For the Coating Material: Harmful Substance as coating material to be used for coating the casing, ppm (mg/kg), Max |  |  |  | 14 | 10000.00 | 4 | 20000.00 | 0.00 |
|  |  | i) Antimony (Sb) | 45.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | ii) Arsenic (As) | 3.80 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | iii) Barium (Ba) | 1500 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | iv) Cadmium (Cd) | 1.30 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | v) Lead (Pb) | 2.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | vi) Mercury (Hg) | 7.50 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | vii) Selenium (Se) | 37.50 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | viii) Chromium (CrIII) | 37.50 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | ix) Chromium (CrVI) | 0.02 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | x) Chromium (Cr) | --- | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | b)For the Colour Pencil Lead: Hazardous Substances, ppm (mg/kg), Max |  |  |  | 14 |  | 4 |  | 0.00 |
|  |  | i) Antimony (Sb) | 45.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | ii) Arsenic (As) | 3.80 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | iii) Barium (Ba) | 1500.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | iv) Cadmium (Cd) | 1.30 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | v) Lead (Pb) | 2.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | vi) Mercury (Hg) | 7.50 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | vii) Selenium (Se) | 37.50 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | viii) Chromium (CrIII) | 37.50 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | ix) Chromium (CrVI) | 0.02 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | x) Chromium (Cr) | ---- | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  | 95.c) Colour Pencils (Type 2: Soft, Diameter of leads, 3.5 mm (Min) (BDS 2020:2023) | a) For the Coating Material: Harmful Substance as coating material to be used for coating the casing, ppm (mg/kg), Max |  |  |  | 14 | 10000.00 | 4 | 20000.00 | 0.00 |
|  |  | i) Antimony (Sb) | 45.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | ii) Arsenic (As) | 3.80 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | iii) Barium (Ba) | 1500 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | iv) Cadmium (Cd) | 1.30 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | v) Lead (Pb) | 2.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | vi) Mercury (Hg) | 7.50 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | vii) Selenium (Se) | 37.50 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | viii) Chromium (CrIII) | 37.50 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | ix) Chromium (CrVI) | 0.02 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | x) Chromium (Cr) | --- | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | b)For the Colour Pencil Lead: Hazardous Substances, ppm (mg/kg), Max |  |  |  | 14 |  | 4 |  | 0.00 |
|  |  | i) Antimony (Sb) | 45.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | ii) Arsenic (As) | 3.80 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | iii) Barium (Ba) | 1500.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | iv) Cadmium (Cd) | 1.30 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | v) Lead (Pb) | 2.00 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | vi) Mercury (Hg) | 7.50 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | vii) Selenium (Se) | 37.50 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | viii) Chromium (CrIII) | 37.50 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | ix) Chromium (CrVI) | 0.02 | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
|  |  | x) Chromium (Cr) | ---- | BDS ISO 8124-3 | 500.00 | 14 |  | 4 |  | 0.00 |
| 96. | Reusable Sanitary Napkin (BDS 2024:2024) | Total Viable Bacterial Count per Sanitary napkin |  |  | 1000.00 | 14 | 1000.00 | 5 | 2000.00 | 0.00 |
| 97. | 97.Structural Steel and Angels (BDS ISO 630:1,2,3) | Carbon (C), % m/m |  |  | 1000.00 | 14 | 5000.00 | 4 | 10000.00 | 0.00 |
|  |  | Sulphur (S), % m/m |  |  | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Phosphorous (P), % m/m |  |  | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Silicon (Si), %m/m |  |  | 1000.00 | 14 |  | 4 |  | 0.00 |
|  |  | Manganese (Mn), %m/m |  |  | 1000.00 | 14 |  | 4 |  | 0.00 |